# FLASH DRYER

# FAN CALCULATION

TABLE OF CONTENT

1. Centrifugal Fan Calculation3

1.1. Fan Air Volume3

1.2. Pressure Drop4

1.3. Fan Motor Power10

1.4 Summary11

- Centrifugal Fan Calculation

  - Fan Air Volume

The required air volume or flow of the centrifugal fan are calculated in sections 1.3 and 1.4:

Qs=Ma.∆H

Where:

Ma: Mass of air, kg/h

∆H: Change in enthalpy of air: Ho – Hi

Hi: Enthalpy of air (Outlet air temp. 55 ˚C, 43 % Moisture)

Ho: Enthalpy of air (after heater 170 ˚C, 1% Moisture)

Hence:

Ma=ra.Va

ra: Density of air, kg/m3

Va: Volume of air, m3/h

7,739,363,411=Ma.(319768−171430)

Ma=52,174 Kg/h

53,121=0.78×Va

Va=66,890 m3/h

Required Air Volume is: 67000 m3/h

  - Pressure Drop

The total pressure loss of the pneumatic system can be expressed as:

∆Pt=∆Pacc+∆Pg+∆Ps+∆Hg+∆Hs+∆Pmisc

∆Pt: Total pressure loss in the system

∆Pacc: Pressure loss due to acceleration of the solids from their “at rest” condition at the pickup point.

∆Pg: Friction pressure loss of the gas

∆Ps: Friction pressure loss of the solids

∆Hg: Elevation pressure loss of the gas

∆Hs: Elevation pressure loss of the solid

∆Pmisc: Pressure losses from miscellaneous equipment (cyclone, bag filter)

    - Pressure loss due to acceleration of the solids:

∆Pacc=6894.8W.Vp144.g (Pa)

Where:

W: Solids mass velocity, ( lbss.ft2)

Vp: Particle velocity, (ft/s)

g: Acceleration due to gravity, (32.2)

Hence:

Vp=0.8Vg

Vg: Maximum Air velocity, (ft/s) = 65.6

W=MsA

Ms: Starch feed rate, (lbs/s)

A: Duct Area, ( ft2)

W=0.454lbss.ft2

Vp=52 fts

∆Pacc=6894.80.454×52144×32.2=35.4 (Pa)

    - Friction Pressure loss of the gas:

The frictional pressure drop of the gas is calculated based on the following diagram:

∆Pg=Fl.L (Pa)

Where:

Fl: Friction loss, from chart, (Pa/m)

L: Pipes equivalent length, (m) (From 1.2.3)

Figure -1

Q=90000  m3h=52,950 CFM

Duct Size =1300 mm=51 in

Fl=Friction Loss =0.2 Inch H2O100 ft=1.64 Pa/m

L =282 m    (From 1.2.3)

∆Pg=Fl.L=1.64×282=463 (Pa)

    - Pipes equivalent length:

For straight runs use the actual length of the pipe. For bends and other devices, use the table-1 below as a guide:

Table-1

Figure -2

Table -2 (Duct Equivalent Calculation)

Safety Factor =2

Total Duct Length =2 ×141

Total Duct Length =282 m

    - Friction pressure loss of the solids:

∆Ps=∆Pg.K.R   (Pa)

Where:

∆Pg: Friction pressure loss of the gas (Pa)(From 1.2.2)

K: Friction multiplier for the solids conveyed=10

R: Solids to gas mass flow ratio, (lb/lb)

Hence:R=WVp.ρ

W: Solids mass velocity, lbss.ft2

Vp: Particle velocity, ft/s

ρ: Air density (lb/ft3)

W: Starch Feed Rate / Duct Area, lbss.ft2

W=10600×2.236003.14× 4.2724=0.454

Vp=0.8×20×3.28=52

ρ=0.048

R=WVp.ρ=0.45452×0.048=0.179

∆Ps=463×10×0.179=828.77  (Pa)

    - Elevation pressure loss of the gas:

∆Hg=6894.8∆Z.ρ.g144.gc  (Pa)

Where:

∆Hg: Elevation pressure loss of the gas, (Pa)

∆Z: Elevation change in pipeline, (ft)

ρ: Air density, (lb/ft3)

g:Acceleration  due to gravity=32.2

gc:Constant=32.174

∆Z=30 m=98 ft

ρ=0.048

∆Hg=6894.8∆Z.ρ.g144.gc=6894.8×98×0.048×32.2144×32.174=226(Pa)

    - Elevation pressure loss of the solids:

∆Hs=6894.8∆Z.W.g144.Vp.gc  (Pa)

Where:

∆Hs: Elevation pressure loss of the solids, (Pa)

∆Z: Elevation change in pipeline, (ft)

W: Solids mass velocity, ( lbss.ft2)

Vp: Particle velocity, (ft/s)

g:Acceleration  due to gravity=32.2

gc:Constant=32.174

∆Hs=6894.8∆Z.W.g144.Vp.gc=6894.8×98×0.454×32.2144×52×32.174=41  (Pa)

The total pressure loss of the pneumatic system can be expressed as:

∆Pt=∆Pacc+∆Pg+∆Ps+∆Hg+∆Hs+∆Pmisc

∆Pmisc=∆P cyclone=1433 Pa

∆Pt=35.4+463+828.7+226+41+1432

=3026 Pa

Safety Factor=2

∆Pt=3027×2=6052  Pa

  - Fan Motor Power

P=S.FVa.∆Pte.3.6×106 (Kw)

Va:Fan Air Volume (m3/h)

∆Pt:Total Pressure Drop (Pa)

S.F:Satety Factor

e:Fan Mechanical and Electrical Efficiancy

Va=67,000 m3/h

∆Pt=6054 Pa

e=0.68

S.F=1.15

P=1.1567,000×60520.68×3.6×106=190 (Kw)

Fan Motor Power =190(Kw)

  - Summary

The summary of calculation is according to below table:

Table -3 (Summary of Calculation)