Table of Contents

Cover Sheet	2

Title Page	3

Warnings and Errors:	4

Input Echo:	5

XY Coordinate Calculations:	14

Internal Pressure Calculations:	15

External Pressure Calculations:	21

Element and Detail Weights:	42

Conical Section:	45

Conical Section:	49

Conical Section:	53

Conical Section:	57

Conical Section:	61

Center of Gravity Calculation:	64

MDMT Summary:	65

Vessel Design Summary:	66

DESIGN CALCULATION

In Accordance with ASME Section VIII Division 1

ASME Code Version     : 2017

Analysis Performed by : SPLM Licensed User

Job File              : D:\2-PAYAM\1-PROJECTS\MICROTEC PROJECT\13-MICROT

Date of Analysis      : Feb 20,2026   4:14pm

PV Elite 2019 SP1, March 2019

Note:

PV Elite performs all calculations internally in Imperial Units

to remain compliant with the ASME Code and any built in assumptions

in the ASME Code formulas. The finalized results are reflected to show

the user's set of selected units.

Class From To : Basic Element Checks.

==========================================================================

Class From To: Check of Additional Element Data

==========================================================================

There were no geometry errors or warnings.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

PV Elite Vessel Analysis Program: Input Data

Design Internal Pressure (for Hydrotest)                 0.5  bars

Design Internal Temperature                            170.0  °C

Type of Hydrotest                              not Specified

Hydrotest Position                                Horizontal

Projection of Nozzle from Vessel Top                       0  mm.

Projection of Nozzle from Vessel Bottom                    0  mm.

Minimum Design Metal Temperature                       -28.9  °C

Type of Construction                                  Welded

Special Service                                         None

Degree of Radiography                                   RT-1

Use Higher Longitudinal Stresses (Flag)                    Y

Select t for Internal Pressure (Flag)                      N

Select t for External Pressure (Flag)                      N

Select t for Axial Stress (Flag)                           N

Select Location for Stiff. Rings (Flag)                    N

Consider Vortex Shedding                                   N

Perform a Corroded Hydrotest                               N

Load Case 1                                   NP+EW+WI+FW+BW

Load Case 2                                   NP+EW+EE+FS+BS

Load Case 3                                   NP+OW+WI+FW+BW

Load Case 4                                   NP+OW+EQ+FS+BS

Load Case 5                                         NP+HW+HI

Load Case 6                                         NP+HW+HE

Load Case 7                                   IP+OW+WI+FW+BW

Load Case 8                                   IP+OW+EQ+FS+BS

Load Case 9                                   EP+OW+WI+FW+BW

Load Case 10                                  EP+OW+EQ+FS+BS

Load Case 11                                        HP+HW+HI

Load Case 12                                        HP+HW+HE

Load Case 13                                        IP+WE+EW

Load Case 14                                        IP+WF+CW

Load Case 15                                        IP+VO+OW

Load Case 16                                        IP+VE+EW

Load Case 17                                        NP+VO+OW

Load Case 18                                     FS+BS+IP+OW

Load Case 19                                     FS+BS+EP+OW

Wind Design Code                                   ASCE-7 93

Basic Wind Speed                     [V]              112.65  Km/hr

Surface Roughness Category                   C: Open Terrain

Importance Factor                                        1.0

Type of Surface                            Moderately Smooth

Base Elevation                                             0  mm.

Percent Wind for Hydrotest                              33.0

Using User defined Wind Press. Vs Elev.                    N

Damping Factor (Beta) for Wind (Ope)                  0.0100

Damping Factor (Beta) for Wind (Empty)                0.0000

Damping Factor (Beta) for Wind (Filled)               0.0000

Seismic Design Code                                   UBC 94

UBC Seismic Zone (1=1,2=2a,3=2b,4=3,5=4)               0.000

UBC Importance Factor                                  1.000

UBC Soil Type                                          S1

UBC Horizontal Force Factor                            3.000

UBC Percent Seismic for Hydrotest                      0.000

Design Pressure + Static Head                              Y

Consider MAP New and Cold in Noz. Design                   N

Consider External Loads for Nozzle Des.                    Y

Use ASME VIII-1 Appendix 1-9                               N

Material Database Year        Current w/Addenda or Code Year

Configuration Directives:

Do not use Nozzle MDMT Interpretation VIII-1 01-37        No

Use Table G instead of exact equation for "A"            Yes

Shell Head Joints are Tapered                            Yes

Compute "K" in corroded condition                        Yes

Use Code Case 2286                                        No

Use the MAWP to compute the MDMT                         Yes

For thickness ratios <= 0.35, MDMT will be -155F (-104C) Yes

For PWHT & P1 Materials the MDMT can be < -55F (-48C)     No

Using Metric Material Databases, ASME II D                No

Calculate B31.3 type stress for Nozzles with Loads       Yes

Reduce the MDMT due to lower membrane stress             Yes

Consider Longitudinal Stress in MDMT calcs. (Div. 1)      No

Complete Listing of Vessel Elements and Details:

Element From Node                                         10

Element To Node                                           20

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                 8052  mm.

Inside Diameter                                         1300  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Allowable Stress, Ambient                          137.9  N./mm²

Allowable Stress, Operating                       104.16  N./mm²

Allowable Stress, Hydrotest                       179.27  N./mm²

Material Density                                0.008027  kg./cm³

P Number Thickness                                     0  mm.

Yield Stress, Operating                           156.11  N./mm²

External Pressure Chart Name                        HA-2

UNS Number                                        S31600

Product Form                                       Plate

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                  Ring:[1 of 1]

Dist. from "FROM" Node / Offset dist                3958  mm.

Inside Diameter of Ring                             1306  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1400  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                  Ring:[2 of 2]

Dist. from "FROM" Node / Offset dist                5946  mm.

Inside Diameter of Ring                             1306  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1400  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                  Ring:[3 of 3]

Dist. from "FROM" Node / Offset dist                7939  mm.

Inside Diameter of Ring                             1306  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1400  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                  Ring:[4 of 4]

Dist. from "FROM" Node / Offset dist                7952  mm.

Inside Diameter of Ring                             1306  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1400  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         20

Element To Node                                           30

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 2000  mm.

Inside Diameter                                         1300  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1730  mm.

Design Length of Cone                                   2000  mm.

Half Apex Angle of Cone                            6.1357331  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     20

Detail Type                                         Ring

Detail ID                                        Ring 16

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1306  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         30

Element To Node                                           40

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                  800  mm.

Inside Diameter                                         1730  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     30

Detail Type                                         Ring

Detail ID                                          Ring5

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1736  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1830  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         40

Element To Node                                           50

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 1200  mm.

Inside Diameter                                         1730  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1400  mm.

Design Length of Cone                                   1200  mm.

Half Apex Angle of Cone                            7.8290772  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     40

Detail Type                                         Ring

Detail ID                                        Ring:17

Dist. from "FROM" Node / Offset dist                1199  mm.

Inside Diameter of Ring                           1406.3  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     40

Detail Type                                         Ring

Detail ID                                        Ring 21

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1736  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1836  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         50

Element To Node                                           60

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                 4500  mm.

Inside Diameter                                         1400  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     50

Detail Type                                         Ring

Detail ID                                         Ring 6

Dist. from "FROM" Node / Offset dist                  90  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     50

Detail Type                                         Ring

Detail ID                                         Ring 7

Dist. from "FROM" Node / Offset dist                 103  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     50

Detail Type                                         Ring

Detail ID                                         Ring 8

Dist. from "FROM" Node / Offset dist                1513  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     50

Detail Type                                         Ring

Detail ID                                         Ring 9

Dist. from "FROM" Node / Offset dist                2913  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         60

Element To Node                                           70

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 2000  mm.

Inside Diameter                                         1400  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1730  mm.

Design Length of Cone                                   2000  mm.

Half Apex Angle of Cone                            4.7162204  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     60

Detail Type                                         Ring

Detail ID                                        Ring 18

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     60

Detail Type                                         Ring

Detail ID                                        Ring 23

Dist. from "FROM" Node / Offset dist                1999  mm.

Inside Diameter of Ring                           1735.8  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1829.8  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         70

Element To Node                                           80

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                  800  mm.

Inside Diameter                                         1730  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     70

Detail Type                                         Ring

Detail ID                                        Ring 10

Dist. from "FROM" Node / Offset dist                 737  mm.

Inside Diameter of Ring                             1736  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1830  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     70

Detail Type                                         Ring

Detail ID                                        Ring 11

Dist. from "FROM" Node / Offset dist                 750  mm.

Inside Diameter of Ring                             1736  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1830  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         80

Element To Node                                           90

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 1200  mm.

Inside Diameter                                         1730  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1400  mm.

Design Length of Cone                                   1200  mm.

Half Apex Angle of Cone                            7.8290772  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     80

Detail Type                                         Ring

Detail ID                                        Ring 24

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1736  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1829.7  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     80

Detail Type                                         Ring

Detail ID                                        Ring 26

Dist. from "FROM" Node / Offset dist                1199  mm.

Inside Diameter of Ring                           1406.3  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1499.9  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         90

Element To Node                                          100

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                 3500  mm.

Inside Diameter                                         1400  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     90

Detail Type                                         Ring

Detail ID                                        Ring 12

Dist. from "FROM" Node / Offset dist                 375  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     90

Detail Type                                         Ring

Detail ID                                       Ring 13]

Dist. from "FROM" Node / Offset dist                1745  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1500  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                        100

Element To Node                                          110

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 2000  mm.

Inside Diameter                                         1400  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                                950  mm.

Design Length of Cone                                   2000  mm.

Half Apex Angle of Cone                            6.4187856  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                    100

Detail Type                                         Ring

Detail ID                                        Ring 22

Dist. from "FROM" Node / Offset dist                1999  mm.

Inside Diameter of Ring                           956.22  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1057.1  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                    100

Detail Type                                         Ring

Detail ID                                        Ring 28

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                             1406  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1506.8  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

XY Coordinate Calculations:

|    |            |            |             |             |

From     | To |  X (Horiz.)|  Y (Vert.) |  DX (Horiz.)|  DY (Vert.) |

|    |        mm. |        mm. |         mm. |         mm. |

----------------------------------------------------------------------

10     |  20|        ... |       8052 |         ... |        8052 |

20     |  30|        ... |      10052 |         ... |        2000 |

30     |  40|        ... |      10852 |         ... |         800 |

40     |  50|        ... |      12052 |         ... |        1200 |

50     |  60|        ... |      16552 |         ... |        4500 |

60     |  70|        ... |      18552 |         ... |        2000 |

70     |  80|        ... |      19352 |         ... |         800 |

80     |  90|        ... |      20552 |         ... |        1200 |

90     | 100|        ... |      24052 |         ... |        3500 |

100     | 110|        ... |      26052 |         ... |        2000 |

----------------------------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element Thickness, Pressure, Diameter and Allowable Stress :

|    | Int. Press |    Nominal | Total Corr |    Element |  Allowable |

From      | To | + Liq. Hd  |  Thickness |  Allowance |   Diameter |  Stress(SE)|

|    |       bars |        mm. |        mm. |        mm. |     N./mm² |

----------------------------------------------------------------------------------

10      |  20|        0.5 |          3 |        ... |       1300 |     104.16 |

20      |  30|        0.5 |          3 |        ... |       1730 |     104.16 |

30      |  40|        0.5 |          3 |        ... |       1730 |     104.16 |

40      |  50|        0.5 |          3 |        ... |       1730 |     104.16 |

50      |  60|        0.5 |          3 |        ... |       1400 |     104.16 |

60      |  70|        0.5 |          3 |        ... |       1730 |     104.16 |

70      |  80|        0.5 |          3 |        ... |       1730 |     104.16 |

80      |  90|        0.5 |          3 |        ... |       1730 |     104.16 |

90      | 100|        0.5 |          3 |        ... |       1400 |     104.16 |

100      | 110|        0.5 |          3 |        ... |       1400 |     104.16 |

----------------------------------------------------------------------------------

Element Required Thickness and MAWP :

|    |     Design |   M.A.W.P. |     M.A.P. |    Minimum |   Required |

From      | To |   Pressure |   Corroded | New & Cold |  Thickness |  Thickness |

|    |       bars |       bars |       bars |        mm. |        mm. |

----------------------------------------------------------------------------------

10      |  20|        0.5 |    4.79375 |    6.34667 |          3 |        1.5 |

20      |  30|        0.5 |    3.58411 |    4.74517 |          3 |        1.5 |

30      |  40|        0.5 |    3.60472 |    4.77245 |          3 |        1.5 |

40      |  50|        0.5 |    3.57118 |    4.72806 |          3 |        1.5 |

50      |  60|        0.5 |    4.45222 |     5.8945 |          3 |        1.5 |

60      |  70|        0.5 |    3.59254 |    4.75633 |          3 |        1.5 |

70      |  80|        0.5 |    3.60472 |    4.77245 |          3 |        1.5 |

80      |  90|        0.5 |    3.57118 |    4.72806 |          3 |        1.5 |

90      | 100|        0.5 |    4.45222 |     5.8945 |          3 |        1.5 |

100      | 110|        0.5 |    4.42438 |    5.85764 |          3 |        1.5 |

----------------------------------------------------------------------------------

Minimum                          3.571        4.728

MAWP: 3.571 bars, limited by: Cone.

Internal Pressure Calculation Results :

ASME Code, Section VIII Division 1, 2017

Cylindrical Shell From 10 To 20 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*650.0)/(104.16*1.0-0.6*0.5)

= 0.3121 + 0.0000 = 0.3121 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (104.16*1.0*3.0)/(650.0+0.6*3.0)

= 4.794 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(650.0+0.6*3.0)

= 6.347 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(650.0+0.6*3.0))/(1.0*3.0)

= 10.864 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.230 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 20 To 30 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1730.0)/(2*0.9943*(104.16*1.0-0.6*0.5))

= 0.4178 + 0.0000 = 0.4178 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*104.16*1.0*3.0*0.994)/(1730.0+1.2*3.0*0.994)

= 3.584 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9943)/(1730.0+1.2*3.0*0.9943)

= 4.745 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1730.0+1.2*3.0*0.9943))/(2*1.0*3.0*0.9943)

= 14.531 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.230 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

Note: The Pressure at the Large Diameter is used in the TR calculation.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 30 To 40 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*865.0)/(104.16*1.0-0.6*0.5)

= 0.4154 + 0.0000 = 0.4154 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (104.16*1.0*3.0)/(865.0+0.6*3.0)

= 3.605 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(865.0+0.6*3.0)

= 4.772 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(865.0+0.6*3.0))/(1.0*3.0)

= 14.448 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.173 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 40 To 50 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1730.0)/(2*0.9907*(104.16*1.0-0.6*0.5))

= 0.4193 + 0.0000 = 0.4193 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*104.16*1.0*3.0*0.991)/(1730.0+1.2*3.0*0.991)

= 3.571 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9907)/(1730.0+1.2*3.0*0.9907)

= 4.728 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1730.0+1.2*3.0*0.9907))/(2*1.0*3.0*0.9907)

= 14.583 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.214 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 50 To 60 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*700.0)/(104.16*1.0-0.6*0.5)

= 0.3361 + 0.0000 = 0.3361 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (104.16*1.0*3.0)/(700.0+0.6*3.0)

= 4.452 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(700.0+0.6*3.0)

= 5.894 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(700.0+0.6*3.0))/(1.0*3.0)

= 11.697 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.214 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 60 To 70 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1730.0)/(2*0.9966*(104.16*1.0-0.6*0.5))

= 0.4168 + 0.0000 = 0.4168 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*104.16*1.0*3.0*0.997)/(1730.0+1.2*3.0*0.997)

= 3.593 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9966)/(1730.0+1.2*3.0*0.9966)

= 4.756 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1730.0+1.2*3.0*0.9966))/(2*1.0*3.0*0.9966)

= 14.496 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.214 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

Note: The Pressure at the Large Diameter is used in the TR calculation.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 70 To 80 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*865.0)/(104.16*1.0-0.6*0.5)

= 0.4154 + 0.0000 = 0.4154 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (104.16*1.0*3.0)/(865.0+0.6*3.0)

= 3.605 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(865.0+0.6*3.0)

= 4.772 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(865.0+0.6*3.0))/(1.0*3.0)

= 14.448 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.173 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 80 To 90 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1730.0)/(2*0.9907*(104.16*1.0-0.6*0.5))

= 0.4193 + 0.0000 = 0.4193 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*104.16*1.0*3.0*0.991)/(1730.0+1.2*3.0*0.991)

= 3.571 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9907)/(1730.0+1.2*3.0*0.9907)

= 4.728 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1730.0+1.2*3.0*0.9907))/(2*1.0*3.0*0.9907)

= 14.583 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.214 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 90 To 100 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*700.0)/(104.16*1.0-0.6*0.5)

= 0.3361 + 0.0000 = 0.3361 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (104.16*1.0*3.0)/(700.0+0.6*3.0)

= 4.452 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(700.0+0.6*3.0)

= 5.894 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(700.0+0.6*3.0))/(1.0*3.0)

= 11.697 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.214 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 100 To 110 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1400.0)/(2*0.9937*(104.16*1.0-0.6*0.5))

= 0.3383 + 0.0000 = 0.3383 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*104.16*1.0*3.0*0.994)/(1400.0+1.2*3.0*0.994)

= 4.424 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9937)/(1400.0+1.2*3.0*0.9937)

= 5.858 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1400.0+1.2*3.0*0.9937))/(2*1.0*3.0*0.9937)

= 11.771 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.315 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Elements Suitable for Internal Pressure.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

External Pressure Calculation Results :

External Pressure Calculations:

|      |    Section |    Outside |  Corroded |    Factor  |     Factor |

From|   To |     Length |   Diameter | Thickness |       A    |        B   |

|      |        mm. |        mm. |       mm. |            |     N./mm² |

-----------------------------------------------------------------------------

10|  Ring|       3958 |       1306 |         3 |0.46719E-04 |    4.48502 |

Ring|  Ring|       1988 |       1306 |         3 |0.95296E-04 |    9.14839 |

Ring|  Ring|       1993 |       1306 |         3 |0.95050E-04 |    9.12477 |

Ring|  Ring|    12.9998 |       1306 |         3 |  0.0047922 |    91.4355 |

Ring|    20|        100 |       1306 |         3 |     0.0026 |    83.9018 |

20|  Ring|    1679.94 |    1306.03 |         3 | 0.00011239 |    10.7893 |

Ring|    30|     1752.3 |    1736.03 |         3 |0.93196E-04 |    8.94674 |

30|  Ring|    No Calc |        ... |         3 |    No Calc |    No Calc |

Ring|    40|        800 |       1736 |         3 | 0.00021156 |    20.3093 |

40|  Ring|     1752.3 |    1736.06 |         3 |0.92641E-04 |    8.89351 |

Ring|  Ring|    1085.13 |    1736.06 |         3 | 0.00015138 |    14.5319 |

Ring|    50|    0.99994 |    1406.33 |         3 |  0.0041294 |    89.7316 |

50|  Ring|         90 |       1406 |         3 |  0.0029414 |    85.5356 |

Ring|  Ring|         13 |       1406 |         3 |  0.0042047 |    89.9431 |

Ring|  Ring|       1410 |       1406 |         3 |   0.000131 |    12.5764 |

Ring|  Ring|       1400 |       1406 |         3 | 0.00013196 |    12.6681 |

Ring|    60|       1587 |       1406 |         3 | 0.00011601 |    11.1366 |

60|  Ring|    0.99994 |    1406.02 |         3 |   0.004178 |    89.8686 |

Ring|  Ring|    1809.08 |    1735.86 |         3 |0.90550E-04 |    8.69271 |

Ring|    70|    0.99989 |    1736.02 |         3 |  0.0028755 |      85.24 |

70|  Ring|        737 |       1736 |         3 | 0.00023058 |    22.1352 |

Ring|  Ring|         13 |       1736 |         3 |  0.0028938 |    85.3229 |

Ring|    80|         50 |       1736 |         3 |  0.0028938 |    85.3229 |

80|  Ring|    0.99989 |    1736.06 |         3 |  0.0028434 |    85.0923 |

Ring|  Ring|    1085.13 |    1736.06 |         3 | 0.00015138 |    14.5319 |

Ring|    90|    0.99994 |    1406.33 |         3 |  0.0041294 |    89.7316 |

90|  Ring|        375 |       1406 |         3 | 0.00052804 |    50.6919 |

Ring|  Ring|       1370 |       1406 |         3 |  0.0001349 |    12.9498 |

Ring|   100|       1755 |       1406 |         3 | 0.00010461 |    10.0421 |

100|  Ring|    0.99994 |    1406.04 |         3 |  0.0041551 |    89.8045 |

Ring|  Ring|    1679.26 |    1406.04 |         3 | 0.00010837 |    10.4036 |

Ring|   110|    0.99982 |    956.263 |         3 |  0.0083163 |    97.0162 |

-----------------------------------------------------------------------------

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    2.64879 |             0.1 |     0.13736 |

Ring|  Ring|          3 |    1.98494 |             0.1 |     0.28018 |

Ring|  Ring|          3 |    1.98708 |             0.1 |     0.27946 |

Ring|  Ring|          3 |    0.47257 |             0.1 |     2.80031 |

Ring|    20|          3 |    0.57319 |             0.1 |     2.56958 |

20|  Ring|          3 |    1.86049 |             0.1 |     0.32853 |

Ring|    30|          3 |    2.24961 |             0.1 |     0.20495 |

30|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    40|          3 |    1.62103 |             0.1 |     0.46793 |

40|  Ring|          3 |    2.25778 |             0.1 |     0.20299 |

Ring|  Ring|          3 |    1.85593 |             0.1 |     0.33169 |

Ring|    50|          3 |    0.51366 |             0.1 |     2.52828 |

50|  Ring|          3 |    0.56956 |             0.1 |      2.4333 |

Ring|  Ring|          3 |    0.50876 |             0.1 |     2.55869 |

Ring|  Ring|          3 |    1.80715 |             0.1 |     0.35777 |

Ring|  Ring|          3 |    1.80239 |             0.1 |     0.36038 |

Ring|    60|          3 |    1.89031 |             0.1 |     0.31681 |

60|  Ring|          3 |    0.51049 |             0.1 |     2.54788 |

Ring|  Ring|          3 |    2.27117 |             0.1 |     0.19962 |

Ring|    70|          3 |     0.6303 |             0.1 |     1.95727 |

70|  Ring|          3 |    1.56614 |             0.1 |        0.51 |

Ring|  Ring|          3 |    0.62816 |             0.1 |     1.96585 |

Ring|    80|          3 |    0.62816 |             0.1 |     1.96585 |

80|  Ring|          3 |    0.63409 |             0.1 |      1.9422 |

Ring|  Ring|          3 |    1.85593 |             0.1 |     0.33169 |

Ring|    90|          3 |    0.51366 |             0.1 |     2.52828 |

90|  Ring|          3 |    1.04348 |             0.1 |     1.44208 |

Ring|  Ring|          3 |    1.78809 |             0.1 |      0.3684 |

Ring|   100|          3 |    1.96728 |             0.1 |     0.28568 |

100|  Ring|          3 |    0.51198 |             0.1 |     2.53866 |

Ring|  Ring|          3 |    1.94504 |             0.1 |      0.2941 |

Ring|   110|          3 |    0.34821 |             0.1 |     4.03247 |

-----------------------------------------------------------------------

Minimum                                                      0.137

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|            3958 |          5366.66 |      No Calc |      No Calc |

Ring|  Ring|            1988 |          5367.77 |      4.74696 |      17.6358 |

Ring|  Ring|            1993 |          5367.78 |      3.17821 |      17.6358 |

Ring|  Ring|         12.9998 |          362.283 |      1.60148 |      17.6358 |

Ring|    20|             100 |          3105.85 |     0.090213 |      17.6358 |

20|  Ring|         1679.94 |          1679.94 |      No Calc |      No Calc |

Ring|    30|          1752.3 |           1752.3 |      2.74012 |      118.645 |

30|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    40|             800 |          3576.41 |      1.50003 |      18.5486 |

40|  Ring|          1752.3 |           1752.3 |      No Calc |      No Calc |

Ring|  Ring|         1085.13 |          1085.13 |      5.32031 |      21.7628 |

Ring|    50|         0.99994 |          0.99994 |      1.08257 |      17.7314 |

50|  Ring|              90 |          2782.45 |      No Calc |      No Calc |

Ring|  Ring|              13 |           332.63 |       0.1026 |      17.8679 |

Ring|  Ring|            1410 |          4840.94 |      1.41706 |      17.8679 |

Ring|  Ring|            1400 |          4840.92 |      2.80393 |      17.8679 |

Ring|    60|            1587 |          4841.33 |      2.96283 |      17.8679 |

60|  Ring|         0.99994 |          0.99994 |      No Calc |      No Calc |

Ring|  Ring|         1809.08 |          1809.08 |      1.80308 |      17.8679 |

Ring|    70|         0.99989 |          0.99989 |      3.39301 |      18.5482 |

70|  Ring|             737 |          3576.31 |      No Calc |      No Calc |

Ring|  Ring|              13 |          979.513 |      1.41118 |      18.5486 |

Ring|    80|              50 |          1799.98 |      0.11813 |      18.5486 |

80|  Ring|         0.99989 |          0.99989 |      No Calc |      No Calc |

Ring|  Ring|         1085.13 |          1085.13 |       2.0298 |       18.381 |

Ring|    90|         0.99994 |          0.99994 |      1.08257 |      17.7021 |

90|  Ring|             375 |          4840.98 |      No Calc |      No Calc |

Ring|  Ring|            1370 |          4840.85 |      1.73585 |      17.8679 |

Ring|   100|            1755 |          4841.66 |      3.11292 |      17.8679 |

100|  Ring|         0.99994 |          0.99994 |      No Calc |      No Calc |

Ring|  Ring|         1679.26 |          1679.26 |      1.67377 |      21.4269 |

Ring|   110|         0.99982 |          0.99982 |      0.52653 |      20.0339 |

--------------------------------------------------------------------------------

Elements Suitable for External Pressure.

ASME Code, Section VIII Division 1, 2017

Cylindrical Shell From 10 to Ring:[1 of 1] Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     3958.00    435.33    3.0306  0.0000467     4.49

EMAP = (4*B)/(3*(D/t)) = (4*4.485)/(3*435.3333) = 0.1374 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.649    1306.00     3958.00    493.06    3.0306  0.0000385     3.70

EMAP = (4*B)/(3*(D/t)) = (4*3.6983)/(3*493.0556) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     5366.66    435.33    4.1092  0.0000340     3.27

EMAP = (4*B)/(3*(D/t)) = (4*3.268)/(3*435.3333) = 0.1001 bars

Cylindrical Shell From Ring:[1 of 1] to Ring:[2 of 2] Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     1988.00    435.33    1.5222  0.0000953     9.15

EMAP = (4*B)/(3*(D/t)) = (4*9.1484)/(3*435.3333) = 0.2802 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.985    1306.00     1988.00    657.96    1.5222  0.0000514     4.94

EMAP = (4*B)/(3*(D/t)) = (4*4.9351)/(3*657.9558) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     5367.77    435.33    4.1101  0.0000340     3.27

EMAP = (4*B)/(3*(D/t)) = (4*3.2673)/(3*435.3333) = 0.1001 bars

Cylindrical Shell From Ring:[2 of 2] to Ring:[3 of 3] Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     1993.00    435.33    1.5260  0.0000951     9.12

EMAP = (4*B)/(3*(D/t)) = (4*9.1248)/(3*435.3333) = 0.2795 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.987    1306.00     1993.00    657.25    1.5260  0.0000514     4.93

EMAP = (4*B)/(3*(D/t)) = (4*4.9298)/(3*657.2463) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1306.00     5367.78    435.33    4.1101  0.0000340     3.27

EMAP = (4*B)/(3*(D/t)) = (4*3.2673)/(3*435.3333) = 0.1001 bars

Cone From Ring 16 to 30 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.983    1736.03     1752.30    582.01    1.0094  0.0000932     8.95

EMAP = (4*B)/(3*(D/t)) = (4*8.9467)/(3*582.0123) = 0.2049 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.237    1736.03     1752.30    776.15    1.0094  0.0000606     5.82

EMAP = (4*B)/(3*(D/t)) = (4*5.8216)/(3*776.1512) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cylindrical Shell From Ring5 to 40 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1736.00      800.00    578.67    0.4608  0.0002116    20.31

EMAP = (4*B)/(3*(D/t)) = (4*20.3093)/(3*578.6667) = 0.4679 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.621    1736.00      800.00   1070.92    0.4608  0.0000837     8.03

EMAP = (4*B)/(3*(D/t)) = (4*8.0326)/(3*1070.9225) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1736.00     3576.41    578.67    2.0601  0.0000452     4.34

EMAP = (4*B)/(3*(D/t)) = (4*4.3427)/(3*578.6667) = 0.1001 bars

Cone From Ring 21 to Ring:17 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.972    1736.06     1085.13    584.13    0.6251  0.0001514    14.53

EMAP = (4*B)/(3*(D/t)) = (4*14.5319)/(3*584.1302) = 0.3317 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.839    1736.06     1085.13    944.21    0.6251  0.0000738     7.08

EMAP = (4*B)/(3*(D/t)) = (4*7.0822)/(3*944.2127) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring:17 to 50 Ext. Chart: HA-2 at 50 °C

Cylindrical Shell From Ring 7 to Ring 8 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     1410.00    468.67    1.0028  0.0001310    12.58

EMAP = (4*B)/(3*(D/t)) = (4*12.5764)/(3*468.6667) = 0.3578 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.807    1406.00     1410.00    778.02    1.0028  0.0000608     5.84

EMAP = (4*B)/(3*(D/t)) = (4*5.8358)/(3*778.0202) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4840.94    468.67    3.4431  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5182)/(3*468.6667) = 0.1001 bars

Cylindrical Shell From Ring 8 to Ring 9 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     1400.00    468.67    0.9957  0.0001320    12.67

EMAP = (4*B)/(3*(D/t)) = (4*12.6681)/(3*468.6667) = 0.3604 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.802    1406.00     1400.00    780.07    0.9957  0.0000610     5.85

EMAP = (4*B)/(3*(D/t)) = (4*5.8512)/(3*780.0737) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4840.92    468.67    3.4430  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5182)/(3*468.6667) = 0.1001 bars

Cylindrical Shell From Ring 9 to 60 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     1587.00    468.67    1.1287  0.0001160    11.14

EMAP = (4*B)/(3*(D/t)) = (4*11.1366)/(3*468.6667) = 0.3168 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.890    1406.00     1587.00    743.79    1.1287  0.0000581     5.58

EMAP = (4*B)/(3*(D/t)) = (4*5.5789)/(3*743.7935) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4841.33    468.67    3.4433  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5179)/(3*468.6667) = 0.1001 bars

Cone From Ring 18 to Ring 23 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.990    1735.86     1809.08    580.58    1.0422  0.0000905     8.69

EMAP = (4*B)/(3*(D/t)) = (4*8.6927)/(3*580.5842) = 0.1996 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.263    1735.86     1809.08    766.90    1.0422  0.0000599     5.75

EMAP = (4*B)/(3*(D/t)) = (4*5.7522)/(3*766.8951) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 23 to 70 Ext. Chart: HA-2 at 50 °C

Cylindrical Shell From 70 to Ring 10 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1736.00      737.00    578.67    0.4245  0.0002306    22.14

EMAP = (4*B)/(3*(D/t)) = (4*22.1352)/(3*578.6667) = 0.51 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.566    1736.00      737.00   1108.46    0.4245  0.0000866     8.31

EMAP = (4*B)/(3*(D/t)) = (4*8.3142)/(3*1108.459) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1736.00     3576.31    578.67    2.0601  0.0000452     4.34

EMAP = (4*B)/(3*(D/t)) = (4*4.3428)/(3*578.6667) = 0.1001 bars

Cone From Ring 24 to Ring 26 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.972    1736.06     1085.13    584.13    0.6251  0.0001514    14.53

EMAP = (4*B)/(3*(D/t)) = (4*14.5319)/(3*584.1302) = 0.3317 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.839    1736.06     1085.13    944.21    0.6251  0.0000738     7.08

EMAP = (4*B)/(3*(D/t)) = (4*7.0822)/(3*944.2127) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 26 to 90 Ext. Chart: HA-2 at 50 °C

Cylindrical Shell From 90 to Ring 12 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00      375.00    468.67    0.2667  0.0005280    50.69

EMAP = (4*B)/(3*(D/t)) = (4*50.6919)/(3*468.6667) = 1.4421 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.043    1406.00      375.00   1347.41    0.2667  0.0001053    10.11

EMAP = (4*B)/(3*(D/t)) = (4*10.1068)/(3*1347.4142) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4840.98    468.67    3.4431  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5182)/(3*468.6667) = 0.1001 bars

Cylindrical Shell From Ring 12 to Ring 13] Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     1370.00    468.67    0.9744  0.0001349    12.95

EMAP = (4*B)/(3*(D/t)) = (4*12.9498)/(3*468.6667) = 0.3684 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.788    1406.00     1370.00    786.31    0.9744  0.0000614     5.90

EMAP = (4*B)/(3*(D/t)) = (4*5.898)/(3*786.3148) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4840.85    468.67    3.4430  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5183)/(3*468.6667) = 0.1001 bars

Cylindrical Shell From Ring 13] to 100 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     1755.00    468.67    1.2482  0.0001046    10.04

EMAP = (4*B)/(3*(D/t)) = (4*10.0421)/(3*468.6667) = 0.2857 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.967    1406.00     1755.00    714.69    1.2482  0.0000558     5.36

EMAP = (4*B)/(3*(D/t)) = (4*5.3608)/(3*714.6927) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1406.00     4841.66    468.67    3.4436  0.0000366     3.52

EMAP = (4*B)/(3*(D/t)) = (4*3.5177)/(3*468.6667) = 0.1001 bars

Cone From Ring 28 to Ring 22 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.981    1406.04     1679.26    471.64    1.1943  0.0001084    10.40

EMAP = (4*B)/(3*(D/t)) = (4*10.4036)/(3*471.6358) = 0.2941 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.933    1406.04     1679.26    727.44    1.1943  0.0000568     5.46

EMAP = (4*B)/(3*(D/t)) = (4*5.4564)/(3*727.4449) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 22 to the end: Ext. Chart: HA-2 at 50 °C

Stiffening Ring Calcs for : Ring:[1 of 1] , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            69 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.066          1.5000           3.098

Ring :             4.700         26.5000         124.550

Total:             6.766                         127.648

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         17.3673         623.030

Ring :             8.652         -7.6327         273.815

Total:             8.667                         896.845

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.10 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000323

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1306.0²*2973.0002(3.0+4.7/2973.0002)0.000032)/10.9

= 5 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      3.03  Kgf/mm.

The Radial Shear Load                             V     39.59  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3587.38  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.81  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.59  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     45.03  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      3.14  Kgf/mm.

Stiffening Ring Calcs for : Ring:[2 of 2] , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            69 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.066          1.5000           3.098

Ring :             4.700         26.5000         124.550

Total:             6.766                         127.648

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         17.3673         623.030

Ring :             8.652         -7.6327         273.815

Total:             8.667                         896.845

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.03 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000315

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1306.0²*1990.5(3.0+4.7/1990.5)0.000032)/10.9

= 3 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      2.03  Kgf/mm.

The Radial Shear Load                             V     26.51  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3587.38  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.54  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.59  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     45.03  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      2.10  Kgf/mm.

Stiffening Ring Calcs for : Ring:[3 of 3] , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            69 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.066          1.5000           3.098

Ring :             4.700         26.5000         124.550

Total:             6.766                         127.648

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         17.3673         623.030

Ring :             8.652         -7.6327         273.815

Total:             8.667                         896.845

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.82 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000294

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1306.0²*1003.0(3.0+4.7/1003.0)0.000029)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.02  Kgf/mm.

The Radial Shear Load                             V     13.36  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3587.38  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.27  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.59  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     45.03  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.06  Kgf/mm.

Stiffening Ring Calcs for : Ring:[4 of 4] , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            69 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.066          1.5000           3.098

Ring :             4.700         26.5000         124.550

Total:             6.766                         127.648

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         17.3673         623.030

Ring :             8.652         -7.6327         273.815

Total:             8.667                         896.845

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        0.87 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000090

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1306.0²*56.5(3.0+4.7/56.5)0.000009)/10.9

= 0 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.06  Kgf/mm.

The Radial Shear Load                             V      0.75  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3587.38  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.02  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.59  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     45.03  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.06  Kgf/mm.

Stiffening Ring Calcs for : Ring 16 , SA-240 304 , Bar Ring: 97 x 10 mm.

Effective Length of Shell:                            69 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.066          1.5000           3.098

Ring :             9.700         51.5000         499.550

Total:            11.766                         502.648

--------------------------------------------------------

Centroid of Ring plus Shell:                          43 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         41.2219        3509.954

Ring :            76.056         -8.7781         747.439

Total:            76.071                        4257.393

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:        119 cm**4

Required Stress in Ring plus Shell      Breq        2.75 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000286

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1306.0²*1716.1235(3.0+9.7/1716.1235)0.000029)/10.9

= 3 cm**4

Stiffening Ring Calcs for : Ring5 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             4.700         26.5000         124.550

Total:             7.081                         128.122

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         16.5926         655.656

Ring :             8.652         -8.4075         332.221

Total:             8.670                         987.878

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         19 cm**4

Required Stress in Ring plus Shell      Breq        3.12 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000325

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1736.0²*400.0(3.0+4.7/400.0)0.000032)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.41  Kgf/mm.

The Radial Shear Load                             V      7.08  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3951.51  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.15  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.43  Kgf/mm.

Stiffening Ring Calcs for : Ring 21 , SA-240 304 , Bar Ring: 50 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             5.000         28.0000         140.000

Total:             7.381                         143.572

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         17.9503         767.347

Ring :            10.417         -8.5497         365.487

Total:            10.434                        1132.833

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         22 cm**4

Required Stress in Ring plus Shell      Breq        3.88 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000405

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1736.0²*1418.7195(3.0+5.0/1418.7195)0.00004)/10.9

= 5 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.45  Kgf/mm.

The Radial Shear Load                             V     25.12  Kgf

The First Moment of the Area ( Ring + Shell )     Q   4274.85  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.49  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.53  Kgf/mm.

Stiffening Ring Calcs for : Ring:17 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.686         26.4300         123.851

Total:             6.829                         127.066

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1057         627.176

Ring :             8.575         -7.8243         286.878

Total:             8.591                         914.053

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.73 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000284

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.275²*543.0673(3.0+4.686/543.0673)0.000028)/10.9

= 1 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.55  Kgf/mm.

The Radial Shear Load                             V      7.79  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3666.48  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.16  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.58  Kgf/mm.

Stiffening Ring Calcs for : Ring 6 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        0.87 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000091

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*51.5(3.0+4.7/51.5)0.000009)/10.9

= 0 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.05  Kgf/mm.

The Radial Shear Load                             V      0.74  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.02  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.05  Kgf/mm.

Stiffening Ring Calcs for : Ring 7 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.88 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000300

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*711.5(3.0+4.7/711.5)0.00003)/10.9

= 1 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.73  Kgf/mm.

The Radial Shear Load                             V     10.20  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.21  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.76  Kgf/mm.

Stiffening Ring Calcs for : Ring 8 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.16 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000330

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*1405.0(3.0+4.7/1405.0)0.000033)/10.9

= 3 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.43  Kgf/mm.

The Radial Shear Load                             V     20.14  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.41  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.49  Kgf/mm.

Stiffening Ring Calcs for : Ring 9 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.18 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000330

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*1493.5001(3.0+4.7/1493.5001)0.000033)/10.9

= 3 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.52  Kgf/mm.

The Radial Shear Load                             V     21.41  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.44  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.59  Kgf/mm.

Stiffening Ring Calcs for : Ring 18 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.00 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000312

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*905.0399(3.0+4.7/905.0399)0.000031)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.92  Kgf/mm.

The Radial Shear Load                             V     12.98  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.27  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.96  Kgf/mm.

Stiffening Ring Calcs for : Ring 23 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             4.700         26.5000         124.550

Total:             7.081                         128.122

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         16.5928         655.646

Ring :             8.652         -8.4072         332.200

Total:             8.670                         987.847

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         19 cm**4

Required Stress in Ring plus Shell      Breq        3.70 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000385

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1735.8351²*905.0399(3.0+4.7/905.0399)0.000039)/10.9

= 3 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.92  Kgf/mm.

The Radial Shear Load                             V     16.02  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3951.38  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.34  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.98  Kgf/mm.

Stiffening Ring Calcs for : Ring 10 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             4.700         26.5000         124.550

Total:             7.081                         128.122

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         16.5926         655.656

Ring :             8.652         -8.4075         332.221

Total:             8.670                         987.878

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         19 cm**4

Required Stress in Ring plus Shell      Breq        3.06 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000320

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1736.0²*375.0(3.0+4.7/375.0)0.000032)/10.9

= 1 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.38  Kgf/mm.

The Radial Shear Load                             V      6.64  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3951.51  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.14  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.41  Kgf/mm.

Stiffening Ring Calcs for : Ring 11 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             4.700         26.5000         124.550

Total:             7.081                         128.122

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         16.5926         655.656

Ring :             8.652         -8.4075         332.221

Total:             8.670                         987.878

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         19 cm**4

Required Stress in Ring plus Shell      Breq        0.73 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000076

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1736.0²*31.5(3.0+4.7/31.5)0.000008)/10.9

= 0 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.03  Kgf/mm.

The Radial Shear Load                             V      0.56  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3951.51  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.01  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.03  Kgf/mm.

Stiffening Ring Calcs for : Ring 24 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            79 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.381          1.5000           3.572

Ring :             4.684         26.4175         123.727

Total:             7.065                         127.299

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.018         16.5182         649.795

Ring :             8.561         -8.3993         330.411

Total:             8.579                         980.207

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.37 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000350

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1736.0²*543.0673(3.0+4.6835/543.0673)0.000035)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.55  Kgf/mm.

The Radial Shear Load                             V      9.61  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3933.81  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.21  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.59  Kgf/mm.

Stiffening Ring Calcs for : Ring 26 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.683         26.4150         123.701

Total:             6.826                         126.916

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.0919         626.168

Ring :             8.558         -7.8231         286.600

Total:             8.574                         912.769

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.73 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000284

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.275²*543.0673(3.0+4.683/543.0673)0.000028)/10.9

= 1 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.55  Kgf/mm.

The Radial Shear Load                             V      7.79  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3663.53  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.16  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.58  Kgf/mm.

Stiffening Ring Calcs for : Ring 12 , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.98 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000310

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*872.5(3.0+4.7/872.5)0.000031)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.89  Kgf/mm.

The Radial Shear Load                             V     12.51  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.26  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.93  Kgf/mm.

Stiffening Ring Calcs for : Ring 13] , SA-240 304 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             4.700         26.5000         124.550

Total:             6.843                         127.765

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         17.1703         631.861

Ring :             8.652         -7.8297         288.132

Total:             8.668                         919.992

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.19 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000333

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*1562.5(3.0+4.7/1562.5)0.000033)/10.9

= 3 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.59  Kgf/mm.

The Radial Shear Load                             V     22.40  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3679.97  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.46  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.66  Kgf/mm.

Stiffening Ring Calcs for : Ring 28 , SA-240 304 , Bar Ring: 50 x 10 mm.

Effective Length of Shell:                            71 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.143          1.5000           3.215

Ring :             5.042         28.2100         142.235

Total:             7.185                         145.450

--------------------------------------------------------

Centroid of Ring plus Shell:                          20 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.016         18.7429         752.904

Ring :            10.681         -7.9671         320.040

Total:            10.697                        1072.945

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         21 cm**4

Required Stress in Ring plus Shell      Breq        2.93 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000305

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1406.0²*840.1315(3.0+5.042/840.1315)0.000031)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.86  Kgf/mm.

The Radial Shear Load                             V     12.05  Kgf

The First Moment of the Area ( Ring + Shell )     Q   4017.02  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.23  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.89  Kgf/mm.

Stiffening Ring Calcs for : Ring 22 , SA-240 304 , Bar Ring: 50 x 10 mm.

Effective Length of Shell:                            59 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.767          1.5000           2.651

Ring :             5.042         28.2113         142.248

Total:             6.810                         144.899

--------------------------------------------------------

Centroid of Ring plus Shell:                          21 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.013         19.7783         691.404

Ring :            10.683         -6.9330         242.361

Total:            10.696                         933.765

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         20 cm**4

Required Stress in Ring plus Shell      Breq        1.99 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000208

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (956.225²*840.1315(3.0+5.0423/840.1315)0.000021)/10.9

= 1 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.86  Kgf/mm.

The Radial Shear Load                             V      8.19  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3495.77  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.14  Kgf/mm.

The Weld Allowable Stress                    0.55*S     73.09  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     44.72  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.87  Kgf/mm.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element and Detail Weights:

|    |    Element |    Element |  Corroded |   Corroded | Extra due  |

From| To | Metal Wgt. | ID  Volume |Metal Wgt. |  ID Volume | Misc %     |

|    |        kg. |        Cm. |       kg. |        Cm. |        kg. |

---------------------------------------------------------------------------

10|  20|    793.769 |   10689519 |   793.769 |   10689519 |        ... |

20|  30|    231.016 |    3630184 |   231.016 |    3630184 |        ... |

30|  40|    104.889 |    1880831 |   104.889 |    1880831 |        ... |

40|  50|    143.693 |    2317308 |   143.693 |    2317308 |        ... |

50|  60|    477.657 |    6928454 |   477.657 |    6928454 |        ... |

60|  70|     238.06 |    3862180 |    238.06 |    3862180 |        ... |

70|  80|    104.889 |    1880831 |   104.889 |    1880831 |        ... |

80|  90|    143.693 |    2317308 |   143.693 |    2317308 |        ... |

90| 100|    371.509 |    5388796 |   371.509 |    5388796 |        ... |

100| 110|    179.371 |    2195581 |   179.371 |    2195581 |        ... |

---------------------------------------------------------------------------

Total |       2788 |41090996.00 |      2788 |41090996.00 |          0 |

Weight of Details:

|    |  Weight of |  X Offset, | Y Offset, |

From|Type|     Detail | Dtl. Cent. |Dtl. Cent. |   Description

|    |        kg. |        mm. |       mm. |

-------------------------------------------------

10|Ring|    16.0367 |        ... |      3958 |   Ring:[1 of 1]

10|Ring|    16.0367 |        ... |      5946 |   Ring:[2 of 2]

10|Ring|    16.0367 |        ... |      7939 |   Ring:[3 of 3]

10|Ring|    16.0367 |        ... |      7952 |   Ring:[4 of 4]

20|Ring|    34.3201 |        ... |       ... |   Ring 16

30|Ring|    21.1334 |        ... |       ... |   Ring5

40|Ring|    17.1723 |        ... |      1199 |   Ring:17

40|Ring|    22.5201 |        ... |       ... |   Ring 21

50|Ring|     17.222 |        ... |        90 |   Ring 6

50|Ring|     17.222 |        ... |       103 |   Ring 7

50|Ring|     17.222 |        ... |      1513 |   Ring 8

50|Ring|     17.222 |        ... |      2913 |   Ring 9

60|Ring|     17.222 |        ... |       ... |   Ring 18

60|Ring|    21.1315 |        ... |      1999 |   Ring 23

70|Ring|    21.1334 |        ... |       737 |   Ring 10

70|Ring|    21.1334 |        ... |       750 |   Ring 11

80|Ring|    21.0573 |        ... |       ... |   Ring 24

80|Ring|     17.161 |        ... |      1199 |   Ring 26

90|Ring|     17.222 |        ... |       375 |   Ring 12

90|Ring|     17.222 |        ... |      1745 |   Ring 13]

100|Ring|    12.8003 |        ... |      1999 |   Ring 22

100|Ring|    18.5186 |        ... |       ... |   Ring 28

-------------------------------------------------

Total Weight of Each Detail Type:

Stiffeners                        412.8

-------------------------------------------

Sum of the Detail Weights         412.8 kg.

Weight Summation Results: (kg.)

| Fabricated |  Shop Test |   Shipping |    Erected |      Empty |  Operating |

---------------------------------------------------------------------------------------------

Main Elements |     2788.5 |     2788.5 |     2788.5 |     2788.5 |     2788.5 |     2788.5 |

Stif. Rings |      412.8 |      412.8 |      412.8 |      412.8 |      412.8 |      412.8 |

Test Liquid |        ... |    41065.9 |        ... |        ... |        ... |        ... |

---------------------------------------------------------------------------------------------

Totals |     3201.3 |    44267.2 |     3201.3 |     3201.3 |     3201.3 |     3201.3 |

Weight Summary:

Fabricated Wt.  - Bare Weight without Removable Internals                  3201.3 kg.

Shop Test Wt.   - Fabricated Weight + Water ( Full )                      44267.2 kg.

Shipping Wt.    - Fab. Weight + removable Intls.+ Shipping App.            3201.3 kg.

Erected  Wt.    - Fab. Wt + or - loose items (trays,platforms etc.)        3201.3 kg.

Ope. Wt. no Liq - Fab. Weight + Internals. + Details + Weights             3201.3 kg.

Operating Wt.   - Empty Weight + Operating Liq. Uncorroded                 3201.3 kg.

Field Test Wt.  - Empty Weight + Water (Full)                             44267.2 kg.

Mass of the Upper 1/3 of the Vertical Vessel                               1107.8 kg.

Outside Surface Areas of Elements:

|    |        Surface |

From| To |           Area |

|    |            cm² |

----------------------------

10|  20|         330367 |

20|  30|        96120.1 |

30|  40|        43630.4 |

40|  50|        59784.7 |

50|  60|         198769 |

60|  70|        99045.5 |

70|  80|        43630.4 |

80|  90|        59784.7 |

90| 100|         154598 |

100| 110|        74674.9 |

----------------------------

Total       1160404.250 cm²

Element and Detail Weights:

| To |  Total Ele.| Total. Ele.|Total. Ele.|  Total Dtl.| Oper. Wgt. |

From| To |  Empty Wgt.|  Oper. Wgt.|Hydro. Wgt.| Offset Mom.| No Liquid  |

|    |        kg. |        kg. |       kg. |      Kg-m. |        kg. |

---------------------------------------------------------------------------

10|  20|    857.916 |    857.916 |   857.916 |        ... |    857.916 |

20|  30|    265.336 |    265.336 |   265.336 |        ... |    265.336 |

30|  40|    126.022 |    126.022 |   126.022 |        ... |    126.022 |

40|  50|    183.386 |    183.386 |   183.386 |        ... |    183.386 |

50|  60|    546.545 |    546.545 |   546.545 |        ... |    546.545 |

60|  70|    276.413 |    276.413 |   276.413 |        ... |    276.413 |

70|  80|    147.156 |    147.156 |   147.156 |        ... |    147.156 |

80|  90|    181.912 |    181.912 |   181.912 |        ... |    181.912 |

90| 100|    405.953 |    405.953 |   405.953 |        ... |    405.953 |

100| 110|    210.689 |    210.689 |   210.689 |        ... |    210.689 |

---------------------------------------------------------------------------

Cumulative Vessel Weight

|    | Cumulative Ope | Cumulative | Cumulative  |

From| To | Wgt. No Liquid | Oper. Wgt. | Hydro. Wgt. |

|    |            kg. |        kg. |         kg. |

-------------------------------------------------------

10|  20|        3201.33 |    3201.33 |     3201.33 |

20|  30|        2343.41 |    2343.41 |     2343.41 |

30|  40|        2078.08 |    2078.08 |     2078.08 |

40|  50|        1952.05 |    1952.05 |     1952.05 |

50|  60|        1768.67 |    1768.67 |     1768.67 |

60|  70|        1222.12 |    1222.12 |     1222.12 |

70|  80|         945.71 |     945.71 |      945.71 |

80|  90|        798.554 |    798.554 |     798.554 |

90| 100|        616.643 |    616.643 |     616.643 |

100| 110|        210.689 |    210.689 |     210.689 |

-------------------------------------------------------

Note: The cumulative operating weights no liquid in the column above

are the cumulative operating weights minus the operating liquid

weight minus any weights absent in the empty condition.

Cumulative Vessel Moment

|    | Cumulative | Cumulative |Cumulative |

From| To | Empty Mom. | Oper. Mom. |Hydro. Mom.|

|    |      Kg-m. |      Kg-m. |     Kg-m. |

-------------------------------------------------

10|  20|        ... |        ... |       ... |

20|  30|        ... |        ... |       ... |

30|  40|        ... |        ... |       ... |

40|  50|        ... |        ... |       ... |

50|  60|        ... |        ... |       ... |

60|  70|        ... |        ... |       ... |

70|  80|        ... |        ... |       ... |

80|  90|        ... |        ... |       ... |

90| 100|        ... |        ... |       ... |

100| 110|        ... |        ... |       ... |

-------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 20 To 30 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    12.7573  mm.

Maximum Centroid Reinforcement Distance Small End    11.0651  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00048

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       6.136  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.187  degrees

Large end actual half apex angle                       6.136  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Large End All. Stress * Large End Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        133.8  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0068*0.4426*868.0*0.107/(134*1.0))*

(1-1/4((0.1*868.0-0.443)/0.443)*(0.187/6.136)

= 0.0302 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 868.0/2 ) + 0.0/(pi(1736.0 - 3.0)) +

0.0/(pi(868.0 - 3.0/2)(868.0 - 3.0/2))

= 0.443 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1736.0 * 3.0 )½ * ( 3.0 + 3.0/0.994 )

= 2.3884 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0302  cm²

Area of reinforcement in shell per App. 1-8(2)        2.3884  cm²

Area of reinforcement in stiffening ring              4.7000  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       46.87  cm²

Force per Unit Length on Shell / Cone Junction          1.55  Kgf/mm.

Actual Buckling Stress associated with this Force       4.23  N./mm²

Material Strain associated with this stress         0.000044

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.438E-04 * 1736.0 * 1736.0 * 46.87/10.9

= 56786.08 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.191    0.0000     0.000        16.2161     0.009   313.119

Con      1.198   -2.1334    -2.555        18.3495     0.027   403.238

Sec      4.700   25.0000   117.500        -8.7840     8.652   362.643

TOT      7.088             114.945                    8.688  1079.001

Centroid of Section        16.2161     Moment of Inertia        19.48

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              19.478  cm**4

Required  Moment of Inertia ( Large End )               5.679  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00048

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                       6.136  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.19226E+11/( 100.3 * 0.18458E+09 ), 1 )

= 1.0386

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 104.2 * 0.18458E+09

= 19225886720.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 650.0( 3.0 - 0.0 ) )

= 61.822 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.04 * 1.6571 * 650.0/( 104 * 1.0 ) *

( 1 - 4.0/6.14 ) 0.1075

= 0.0394 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 650.0/2 ) - 0.0/(pi(1300.0 + 3.0)) +

0.0/(pi(650.0 + 3.0/2)(650.0 + 3.0/2))

= 1.657 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(650.0*3.0)½*(( 3.0- 0.312 )+( 3.0- 0.314 )/0.99)

= 1.8563 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0394  cm²

Area of reinforcement in shell per App. 1-5(4)        1.8563  cm²

Area of reinforcement in stiffening ring              9.7000  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         133.8  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0068*0.333*653.0*0.1075/(134*1.0))

= 0.0172 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 653.0/2 ) + 0.0/(pi(1306.0 - 3.0)) +

0.0/(pi(653.0 - 3.0/2)(653.0 - 3.0/2))

= 0.333 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(1306.0*3.0)½*[(3.0-0.573)+(3.0-1.893)/0.994]

= 1.2188 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0172  cm²

Area of reinforcement in shell per App. 1-8(4)        1.2188  cm²

Area of reinforcement in stiffening ring              9.7000  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       41.37  cm²

Force per Unit Length on Shell / Cone Junction          0.88  Kgf/mm.

Actual Buckling Stress associated with this Force       2.04  N./mm²

Material Strain associated with this stress         0.000021

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.211E-04 * 1306.0 * 1306.0 * 41.37/10.9

= 13689.88 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      1.033    0.0000     0.000        41.3643     0.008  1767.127

Con      1.039    1.8504     1.922        39.5139     0.020  1621.850

Sec      9.700   50.0000   485.000        -8.6357    76.056   723.377

TOT     11.772             486.922                   76.083  4112.354

Centroid of Section        41.3643     Moment of Inertia       117.21

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.1172E+03  cm**4

Required  Moment of Inertia ( Small End )            0.1369E+01  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress           15.45          312.47  Small Cyl. Long.

Compres. Stress           -4.60         -312.47  Small Cyl. Long.

Membrane Stress           16.38         -156.24   Small End Tang.

Tensile  Stress           15.49          312.47 Cone Longitudinal

Compres. Stress           -4.56         -312.47 Cone Longitudinal

Compres  Stress           16.44         -156.24   Cone Tangential

Tensile  Stress           22.60          312.47  Large Cyl. Long.

Compres. Stress           -8.16         -312.47  Large Cyl. Long.

Membrane Stress            5.98         -156.24   Large End Tang.

Tensile  Stress           22.64          312.47 Cone Longitudinal

Compres. Stress           -8.11         -312.47 Cone Longitudinal

Compres  Stress            6.06         -156.24   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         3.584     Thickness due to internal pressure, Cone Large End

MAPnc         4.745     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 40 To 50 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    12.7573  mm.

Maximum Centroid Reinforcement Distance Small End    11.4810  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00048

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       7.829  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.187  degrees

Large end actual half apex angle                       7.829  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        133.8  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0068*0.4426*868.0*0.138/(134*1.0))*

(1-1/4((0.1*868.0-0.443)/0.443)*(0.187/7.829)

= 0.0387 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 868.0/2 ) + 0.0/(pi(1736.0 - 3.0)) +

0.0/(pi(868.0 - 3.0/2)(868.0 - 3.0/2))

= 0.443 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1736.0 * 3.0 )½ * ( 3.0 + 3.0/0.991 )

= 2.3927 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0387  cm²

Area of reinforcement in shell per App. 1-8(2)        2.3927  cm²

Area of reinforcement in stiffening ring              5.0000  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       35.17  cm²

Force per Unit Length on Shell / Cone Junction          1.09  Kgf/mm.

Actual Buckling Stress associated with this Force       3.94  N./mm²

Material Strain associated with this stress         0.000041

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.408E-04 * 1736.0 * 1736.0 * 35.17/10.9

= 39720.57 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.191    0.0000     0.000        17.4794     0.009   363.808

Con      1.202   -2.7288    -3.280        20.2082     0.039   490.842

Sec      5.000   26.5000   132.500        -9.0206    10.417   406.853

TOT      7.393             129.220                   10.465  1261.504

Centroid of Section        17.4794     Moment of Inertia        23.08

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              23.080  cm**4

Required  Moment of Inertia ( Large End )               3.972  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00048

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                       7.829  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.19226E+11/( 100.3 * 0.18458E+09 ), 1 )

= 1.0386

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 104.2 * 0.18458E+09

= 19225886720.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 700.0( 3.0 - 0.0 ) )

= 64.156 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.04 * 1.7846 * 700.0/( 104 * 1.0 ) *

( 1 - 4.0/7.83 ) 0.1375

= 0.0822 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 700.0/2 ) - 0.0/(pi(1400.0 + 3.0)) +

0.0/(pi(700.0 + 3.0/2)(700.0 + 3.0/2))

= 1.785 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(700.0*3.0)½*(( 3.0- 0.336 )+( 3.0- 0.339 )/0.99)

= 1.9122 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0822  cm²

Area of reinforcement in shell per App. 1-5(4)        1.9122  cm²

Area of reinforcement in stiffening ring              4.6860  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         133.8  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0068*0.3584*703.0*0.1375/(134*1.0))

= 0.0256 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 703.0/2 ) + 0.0/(pi(1406.0 - 3.0)) +

0.0/(pi(703.0 - 3.0/2)(703.0 - 3.0/2))

= 0.358 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(1406.0*3.0)½*[(3.0-0.57)+(3.0-0.514)/0.991]

= 1.7647 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0256  cm²

Area of reinforcement in shell per App. 1-8(4)        1.7647  cm²

Area of reinforcement in stiffening ring              4.6860  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       24.21  cm²

Force per Unit Length on Shell / Cone Junction          0.55  Kgf/mm.

Actual Buckling Stress associated with this Force       2.35  N./mm²

Material Strain associated with this stress         0.000024

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.244E-04 * 1406.0 * 1406.0 * 24.21/10.9

= 10712.14 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      1.072    0.0000     0.000        17.4694     0.008   327.033

Con      1.082    2.4558     2.656        15.0136     0.030   243.822

Sec      4.686   24.9300   116.822        -7.4606     8.575   260.827

TOT      6.839             119.478                    8.613   831.682

Centroid of Section        17.4694     Moment of Inertia        16.93

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.1693E+02  cm**4

Required  Moment of Inertia ( Small End )            0.1071E+01  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress           20.16          312.47  Small Cyl. Long.

Compres. Stress           -8.47         -312.47  Small Cyl. Long.

Membrane Stress           19.57         -156.24   Small End Tang.

Tensile  Stress           20.22          312.47 Cone Longitudinal

Compres. Stress           -8.41         -312.47 Cone Longitudinal

Compres  Stress           19.68         -156.24   Cone Tangential

Tensile  Stress           26.87          312.47  Large Cyl. Long.

Compres. Stress          -12.43         -312.47  Large Cyl. Long.

Membrane Stress            3.63         -156.24   Large End Tang.

Tensile  Stress           26.94          312.47 Cone Longitudinal

Compres. Stress          -12.36         -312.47 Cone Longitudinal

Compres  Stress            3.76         -156.24   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         3.571     Thickness due to internal pressure, Cone Large End

MAPnc         4.728     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 60 To 70 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    12.7573  mm.

Maximum Centroid Reinforcement Distance Small End    11.4810  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00048

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       4.716  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.187  degrees

Large end actual half apex angle                       4.716  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        133.8  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0068*0.4426*868.0*0.082/(134*1.0))*

(1-1/4((0.1*868.0-0.443)/0.443)*(0.187/4.716)

= 0.0232 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 868.0/2 ) + 0.0/(pi(1736.0 - 3.0)) +

0.0/(pi(868.0 - 3.0/2)(868.0 - 3.0/2))

= 0.443 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1736.0 * 3.0 )½ * ( 3.0 + 3.0/0.997 )

= 2.3855 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0232  cm²

Area of reinforcement in shell per App. 1-8(2)        2.3855  cm²

Area of reinforcement in stiffening ring              4.7000  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       45.86  cm²

Force per Unit Length on Shell / Cone Junction          1.57  Kgf/mm.

Actual Buckling Stress associated with this Force       4.37  N./mm²

Material Strain associated with this stress         0.000045

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.453E-04 * 1736.0 * 1736.0 * 45.86/10.9

= 57448.63 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.191    0.0000     0.000        16.3070     0.009   316.641

Con      1.195   -1.6373    -1.956        17.9443     0.020   384.720

Sec      4.700   25.0000   117.500        -8.6930     8.652   355.172

TOT      7.086             115.544                    8.681  1056.534

Centroid of Section        16.3070     Moment of Inertia        19.25

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              19.246  cm**4

Required  Moment of Inertia ( Large End )               5.745  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00048

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                       4.716  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.19226E+11/( 100.3 * 0.18458E+09 ), 1 )

= 1.0386

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 104.2 * 0.18458E+09

= 19225886720.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 700.0( 3.0 - 0.0 ) )

= 64.156 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.04 * 1.7846 * 700.0/( 104 * 1.0 ) *

( 1 - 4.0/4.72 ) 0.0825

= 0.0153 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 700.0/2 ) - 0.0/(pi(1400.0 + 3.0)) +

0.0/(pi(700.0 + 3.0/2)(700.0 + 3.0/2))

= 1.785 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(700.0*3.0)½*(( 3.0- 0.336 )+( 3.0- 0.337 )/1.)

= 1.9072 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0153  cm²

Area of reinforcement in shell per App. 1-5(4)        1.9072  cm²

Area of reinforcement in stiffening ring              4.7000  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         133.8  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0068*0.3584*703.0*0.0825/(134*1.0))

= 0.0153 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 703.0/2 ) + 0.0/(pi(1406.0 - 3.0)) +

0.0/(pi(703.0 - 3.0/2)(703.0 - 3.0/2))

= 0.358 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(1406.0*3.0)½*[(3.0-1.89)+(3.0-2.0)/0.997]

= 0.7552 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0153  cm²

Area of reinforcement in shell per App. 1-8(4)        0.7552  cm²

Area of reinforcement in stiffening ring              4.7000  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       58.61  cm²

Force per Unit Length on Shell / Cone Junction          1.60  Kgf/mm.

Actual Buckling Stress associated with this Force       2.82  N./mm²

Material Strain associated with this stress         0.000029

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.292E-04 * 1406.0 * 1406.0 * 58.61/10.9

= 31077.31 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      1.072    0.0000     0.000        17.3925     0.008   324.163

Con      1.075    1.4735     1.584        15.9191     0.016   272.487

Sec      4.700   25.0000   117.500        -7.6075     8.652   272.005

TOT      6.847             119.084                    8.676   868.655

Centroid of Section        17.3925     Moment of Inertia        17.36

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.1736E+02  cm**4

Required  Moment of Inertia ( Small End )            0.3108E+01  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress           14.45          312.47  Small Cyl. Long.

Compres. Stress           -2.76         -312.47  Small Cyl. Long.

Membrane Stress           16.43         -156.24   Small End Tang.

Tensile  Stress           14.47          312.47 Cone Longitudinal

Compres. Stress           -2.74         -312.47 Cone Longitudinal

Compres  Stress           16.47         -156.24   Cone Tangential

Tensile  Stress           19.03          312.47  Large Cyl. Long.

Compres. Stress           -4.59         -312.47  Large Cyl. Long.

Membrane Stress            7.94         -156.24   Large End Tang.

Tensile  Stress           19.05          312.47 Cone Longitudinal

Compres. Stress           -4.56         -312.47 Cone Longitudinal

Compres  Stress            7.99         -156.24   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         3.592     Thickness due to internal pressure, Cone Large End

MAPnc         4.756     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 80 To 90 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    12.7573  mm.

Maximum Centroid Reinforcement Distance Small End    11.4810  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00048

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       7.829  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.187  degrees

Large end actual half apex angle                       7.829  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        133.8  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0068*0.4426*868.0*0.138/(134*1.0))*

(1-1/4((0.1*868.0-0.443)/0.443)*(0.187/7.829)

= 0.0387 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 868.0/2 ) + 0.0/(pi(1736.0 - 3.0)) +

0.0/(pi(868.0 - 3.0/2)(868.0 - 3.0/2))

= 0.443 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1736.0 * 3.0 )½ * ( 3.0 + 3.0/0.991 )

= 2.3927 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0387  cm²

Area of reinforcement in shell per App. 1-8(2)        2.3927  cm²

Area of reinforcement in stiffening ring              4.6835  cm²

Intermediate Results, Large End, External Pressure:

Calculate Cylinder Decay Length:

= 2 * sqrt( RL * ts )

= 2*sqrt(868.0*3.0)

= 102.0588 mm.

Note:  LL is zero since large cylinder length is less than the decay length

Area Available in Cone, Shell, and Reinforcement       22.85  cm²

Force per Unit Length on Shell / Cone Junction          0.68  Kgf/mm.

Actual Buckling Stress associated with this Force       3.78  N./mm²

Material Strain associated with this stress         0.000039

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.392E-04 * 1736.0 * 1736.0 * 22.85/10.9

= 24792.07 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.191    0.0000     0.000        16.0286     0.009   305.921

Con      1.202   -2.7288    -3.280        18.7574     0.039   422.893

Sec      4.684   24.9175   116.701        -8.8889     8.561   370.059

TOT      7.076             113.421                    8.609  1098.873

Centroid of Section        16.0286     Moment of Inertia        19.60

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              19.598  cm**4

Required  Moment of Inertia ( Large End )               2.479  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00048

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                       7.829  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.19226E+11/( 100.3 * 0.18458E+09 ), 1 )

= 1.0386

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 104.2 * 0.18458E+09

= 19225886720.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 700.0( 3.0 - 0.0 ) )

= 64.156 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.04 * 1.7846 * 700.0/( 104 * 1.0 ) *

( 1 - 4.0/7.83 ) 0.1375

= 0.0822 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 700.0/2 ) - 0.0/(pi(1400.0 + 3.0)) +

0.0/(pi(700.0 + 3.0/2)(700.0 + 3.0/2))

= 1.785 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(700.0*3.0)½*(( 3.0- 0.336 )+( 3.0- 0.339 )/0.99)

= 1.9122 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0822  cm²

Area of reinforcement in shell per App. 1-5(4)        1.9122  cm²

Area of reinforcement in stiffening ring              4.6830  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         133.8  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0068*0.3584*703.0*0.1375/(134*1.0))

= 0.0256 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 703.0/2 ) + 0.0/(pi(1406.0 - 3.0)) +

0.0/(pi(703.0 - 3.0/2)(703.0 - 3.0/2))

= 0.358 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(1406.0*3.0)½*[(3.0-1.043)+(3.0-0.514)/0.991]

= 1.5954 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0256  cm²

Area of reinforcement in shell per App. 1-8(4)        1.5954  cm²

Area of reinforcement in stiffening ring              4.6830  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       28.48  cm²

Force per Unit Length on Shell / Cone Junction          0.70  Kgf/mm.

Actual Buckling Stress associated with this Force       2.53  N./mm²

Material Strain associated with this stress         0.000026

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.262E-04 * 1406.0 * 1406.0 * 28.48/10.9

= 13537.53 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      1.072    0.0000     0.000        17.4558     0.008   326.525

Con      1.082    2.4558     2.656        15.0000     0.030   243.382

Sec      4.683   24.9150   116.677        -7.4592     8.558   260.558

TOT      6.836             119.333                    8.596   830.465

Centroid of Section        17.4558     Moment of Inertia        16.90

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.1690E+02  cm**4

Required  Moment of Inertia ( Small End )            0.1354E+01  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress           20.16          312.47  Small Cyl. Long.

Compres. Stress           -8.47         -312.47  Small Cyl. Long.

Membrane Stress           19.57         -156.24   Small End Tang.

Tensile  Stress           20.22          312.47 Cone Longitudinal

Compres. Stress           -8.41         -312.47 Cone Longitudinal

Compres  Stress           19.68         -156.24   Cone Tangential

Tensile  Stress           26.87          312.47  Large Cyl. Long.

Compres. Stress          -12.43         -312.47  Large Cyl. Long.

Membrane Stress            3.63         -156.24   Large End Tang.

Tensile  Stress           26.94          312.47 Cone Longitudinal

Compres. Stress          -12.36         -312.47 Cone Longitudinal

Compres  Stress            3.76         -156.24   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         3.571     Thickness due to internal pressure, Cone Large End

MAPnc         4.728     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 100 To 110 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    11.4810  mm.

Maximum Centroid Reinforcement Distance Small End     9.4670  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00048

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       6.419  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.187  degrees

Large end actual half apex angle                       6.419  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.25813E+11/( 132.9 * 0.19291E+09 ), 1 )

= 1.0068

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 133.8 * 0.19291E+09

= 25812725760.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   133.8  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        133.8  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0068*0.3584*703.0*0.112/(134*1.0))*

(1-1/4((0.1*703.0-0.358)/0.358)*(0.187/6.419)

= 0.0208 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 703.0/2 ) + 0.0/(pi(1406.0 - 3.0)) +

0.0/(pi(703.0 - 3.0/2)(703.0 - 3.0/2))

= 0.358 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1406.0 * 3.0 )½ * ( 3.0 + 3.0/0.994 )

= 2.1500 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0208  cm²

Area of reinforcement in shell per App. 1-8(2)        2.1500  cm²

Area of reinforcement in stiffening ring              5.0420  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       61.56  cm²

Force per Unit Length on Shell / Cone Junction          2.00  Kgf/mm.

Actual Buckling Stress associated with this Force       3.35  N./mm²

Material Strain associated with this stress         0.000035

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.348E-04 * 1406.0 * 1406.0 * 61.56/10.9

= 38820.20 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.072    0.0000     0.000        18.4240     0.008   363.751

Con      1.078   -2.0093    -2.167        20.4333     0.023   450.240

Sec      5.042   26.7100   134.672        -8.2860    10.681   346.172

TOT      7.192             132.505                   10.712  1160.163

Centroid of Section        18.4240     Moment of Inertia        22.31

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              22.314  cm**4

Required  Moment of Inertia ( Large End )               3.882  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress           17.57          312.47  Large Cyl. Long.

Compres. Stress           -5.87         -312.47  Large Cyl. Long.

Membrane Stress            5.24         -156.24   Large End Tang.

Tensile  Stress           17.60          312.47 Cone Longitudinal

Compres. Stress           -5.84         -312.47 Cone Longitudinal

Compres  Stress            5.31         -156.24   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         4.424     Thickness due to internal pressure, Cone Large End

MAPnc         5.857     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Shop/Field Installation Options :

Note : The CG is computed from the first Element From Node

Center of Gravity of Stiffening Rings                     14631.545 mm.

Center of Gravity of Bare Shell New and Cold              13151.735 mm.

Center of Gravity of Bare Shell Corroded                  13151.735 mm.

Vessel CG in the Operating Condition                      13342.544 mm.

Vessel CG in the Fabricated (Shop/Empty) Condition        13342.544 mm.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Minimum Design Metal Temperature Results Summary :

Curve  Basic  Reduced  UG-20(f)  Thickness     Gov      E*   PWHT

Description               MDMT     MDMT      MDMT      ratio     Thk           reqd

Notes           °C       °C        °C                mm.

-----------------------------------------------------------------------------------

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

-----------------------------------------------------------------------------------

Warmest MDMT:             -196

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Notes:

[  ! ] - This was an impact tested material.

[  1] - Governing Nozzle Weld.

[  4] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-c).

[  5] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-b).

[  6] - MDMT Calculations at the Shell/Head Joint.

[  7] - MDMT Calculations for the Straight Flange.

[  8] - Cylinder/Cone/Flange Junction MDMT.

[  9] - Calculations in the Spherical Portion of the Head.

[10] - Calculations in the Knuckle Portion of the Head.

[11] - Calculated (Body Flange) Flange MDMT.

[12] - Calculated Flat Head MDMT per UCS-66.3

[13] - Tubesheet MDMT, shell side, if applicable

[14] - Tubesheet MDMT, tube side, if applicable

[15] - Nozzle Material

[16] - Shell or Head Material

[17] - Impact Testing required

[18] - Impact Testing not required, see UCS-66(b)(3)

[19] - Select a valid hydrotest type to get the UG-20(f) exemption

[20] - Cylinder/Cone Junction MDMT based on Longitudinal Stress considerations

[21] - Bolting Material

UG-84(b)(2) was not considered.

UCS-66(g) was not considered.

UCS-66(i) was not considered.

Notes:

Impact test temps were not entered in and not considered in the analysis.

UCS-66(i) applies to impact tested materials not by specification and

UCS-66(g) applies to materials impact tested per UG-84.1 General Note (c).

The Basic MDMT includes the (30F) PWHT credit if applicable.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

ASME Code, Section VIII Division 1, 2017

Diameter Spec : 1300.000 x 1730.000 x 1400.000 x 1730.000 x 1400.000 mm. ID

Vessel Design Length, Tangent to Tangent          26052.00  mm.

Distance of Bottom Tangent above Grade             8052.00  mm.

Distance of Base above Grade                          0.00  mm.

Specified Datum Line Distance                         0.00  mm.

Internal Design Temperature                            170  °C

Internal Design Pressure                             0.500  bars

External Design Temperature                             50  °C

External Design Pressure                             0.100  bars

Maximum Allowable Working Pressure                   3.571  bars

External Max. Allowable Working Pressure             0.137  bars

Hydrostatic Test Pressure                            0.000  bars

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Wind Design Code                                   ASCE-93

Earthquake Design Code                              UBC-94

Materials of Construction:

Component |                 |       |                        |         |Normal |  Impact |

Type |        Material | Class |              Thickness |   UNS # |  ized |  Tested |

----------------------------------------------------------------------------------------------

Shell |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Cone |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Rings |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Rings |      SA-240 304 |   ... |                    ... |  S30400 |    No |      No |

Normalized is determined based on the UCS-66 material curve selection and Figure UCS-66.

Impact Tested is based on material selection and material data properties.

Element Pressures and MAWP (bars & mm.):

|        Design |         |           |             | Str. |    In |

Element Description |      Pressure |    Ext. |   Element |   Corrosion | Flg. | Creep |

or Type |  + Stat. head |  Press. |   M.A.W.P |   Allowance | Gov. | Range |

--------------------------------------------------------------------------------------------

Cylinder |         0.500 |    0.10 |     4.794 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     3.584 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     3.605 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     3.571 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     4.452 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     3.593 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     3.605 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     3.571 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     4.452 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     4.424 |      0.0000 |  N/A |    No |

Stiffener Ring Specifications:

Elevation  mm.     Selected Type         User Description

3958.00       Bar  47.0 x  10.            Ring:[1 of 1]

5946.00       Bar  47.0 x  10.            Ring:[2 of 2]

7939.00       Bar  47.0 x  10.            Ring:[3 of 3]

7952.00       Bar  47.0 x  10.            Ring:[4 of 4]

8052.00       Bar  97.0 x  10.                  Ring 16

10052.00       Bar  47.0 x  10.                    Ring5

12051.00       Bar  46.9 x  10.                  Ring:17

10852.00       Bar  50.0 x  10.                  Ring 21

12142.00       Bar  47.0 x  10.                   Ring 6

12155.00       Bar  47.0 x  10.                   Ring 7

13565.00       Bar  47.0 x  10.                   Ring 8

14965.00       Bar  47.0 x  10.                   Ring 9

16552.00       Bar  47.0 x  10.                  Ring 18

18551.00       Bar  47.0 x  10.                  Ring 23

19289.00       Bar  47.0 x  10.                  Ring 10

19302.00       Bar  47.0 x  10.                  Ring 11

19352.00       Bar  46.8 x  10.                  Ring 24

20551.00       Bar  46.8 x  10.                  Ring 26

20927.00       Bar  47.0 x  10.                  Ring 12

22297.00       Bar  47.0 x  10.                 Ring 13]

26051.00       Bar  50.4 x  10.                  Ring 22

24052.00       Bar  50.4 x  10.                  Ring 28

Element Types and Properties:

Element |"To" Elev |  Element |  Nominal | Finished | Reqd Thk | Reqd Thk |   Long |   Circ |

|          |   Length |Thickness |Thickness | Internal | External |    Eff |    Eff |

Type |      mm. |      mm. |      mm. |      mm. |      mm. |      mm. |        |        |

------------------------------------------------------------------------------------------------

Cylinder |   8052.0 |   8052.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |  10052.0 |   2000.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |  10852.0 |    800.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |  12052.0 |   1200.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |  16552.0 |   4500.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |  18552.0 |   2000.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |  19352.0 |    800.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |  20552.0 |   1200.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |  24052.0 |   3500.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |  26052.0 |   2000.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    2.64879 |             0.1 |     0.13736 |

Ring|  Ring|          3 |    1.98494 |             0.1 |     0.28018 |

Ring|  Ring|          3 |    1.98708 |             0.1 |     0.27946 |

Ring|  Ring|          3 |    0.47257 |             0.1 |     2.80031 |

Ring|    20|          3 |    0.57319 |             0.1 |     2.56958 |

20|  Ring|          3 |    1.86049 |             0.1 |     0.32853 |

Ring|    30|          3 |    2.24961 |             0.1 |     0.20495 |

30|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    40|          3 |    1.62103 |             0.1 |     0.46793 |

40|  Ring|          3 |    2.25778 |             0.1 |     0.20299 |

Ring|  Ring|          3 |    1.85593 |             0.1 |     0.33169 |

Ring|    50|          3 |    0.51366 |             0.1 |     2.52828 |

50|  Ring|          3 |    0.56956 |             0.1 |      2.4333 |

Ring|  Ring|          3 |    0.50876 |             0.1 |     2.55869 |

Ring|  Ring|          3 |    1.80715 |             0.1 |     0.35777 |

Ring|  Ring|          3 |    1.80239 |             0.1 |     0.36038 |

Ring|    60|          3 |    1.89031 |             0.1 |     0.31681 |

60|  Ring|          3 |    0.51049 |             0.1 |     2.54788 |

Ring|  Ring|          3 |    2.27117 |             0.1 |     0.19962 |

Ring|    70|          3 |     0.6303 |             0.1 |     1.95727 |

70|  Ring|          3 |    1.56614 |             0.1 |        0.51 |

Ring|  Ring|          3 |    0.62816 |             0.1 |     1.96585 |

Ring|    80|          3 |    0.62816 |             0.1 |     1.96585 |

80|  Ring|          3 |    0.63409 |             0.1 |      1.9422 |

Ring|  Ring|          3 |    1.85593 |             0.1 |     0.33169 |

Ring|    90|          3 |    0.51366 |             0.1 |     2.52828 |

90|  Ring|          3 |    1.04348 |             0.1 |     1.44208 |

Ring|  Ring|          3 |    1.78809 |             0.1 |      0.3684 |

Ring|   100|          3 |    1.96728 |             0.1 |     0.28568 |

100|  Ring|          3 |    0.51198 |             0.1 |     2.53866 |

Ring|  Ring|          3 |    1.94504 |             0.1 |      0.2941 |

Ring|   110|          3 |    0.34821 |             0.1 |     4.03247 |

-----------------------------------------------------------------------

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|            3958 |          5366.66 |      No Calc |      No Calc |

Ring|  Ring|            1988 |          5367.77 |      4.74696 |      17.6358 |

Ring|  Ring|            1993 |          5367.78 |      3.17821 |      17.6358 |

Ring|  Ring|         12.9998 |          362.283 |      1.60148 |      17.6358 |

Ring|    20|             100 |          3105.85 |     0.090213 |      17.6358 |

20|  Ring|         1679.94 |          1679.94 |      No Calc |      No Calc |

Ring|    30|          1752.3 |           1752.3 |      2.74012 |      118.645 |

30|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    40|             800 |          3576.41 |      1.50003 |      18.5486 |

40|  Ring|          1752.3 |           1752.3 |      No Calc |      No Calc |

Ring|  Ring|         1085.13 |          1085.13 |      5.32031 |      21.7628 |

Ring|    50|         0.99994 |          0.99994 |      1.08257 |      17.7314 |

50|  Ring|              90 |          2782.45 |      No Calc |      No Calc |

Ring|  Ring|              13 |           332.63 |       0.1026 |      17.8679 |

Ring|  Ring|            1410 |          4840.94 |      1.41706 |      17.8679 |

Ring|  Ring|            1400 |          4840.92 |      2.80393 |      17.8679 |

Ring|    60|            1587 |          4841.33 |      2.96283 |      17.8679 |

60|  Ring|         0.99994 |          0.99994 |      No Calc |      No Calc |

Ring|  Ring|         1809.08 |          1809.08 |      1.80308 |      17.8679 |

Ring|    70|         0.99989 |          0.99989 |      3.39301 |      18.5482 |

70|  Ring|             737 |          3576.31 |      No Calc |      No Calc |

Ring|  Ring|              13 |          979.513 |      1.41118 |      18.5486 |

Ring|    80|              50 |          1799.98 |      0.11813 |      18.5486 |

80|  Ring|         0.99989 |          0.99989 |      No Calc |      No Calc |

Ring|  Ring|         1085.13 |          1085.13 |       2.0298 |       18.381 |

Ring|    90|         0.99994 |          0.99994 |      1.08257 |      17.7021 |

90|  Ring|             375 |          4840.98 |      No Calc |      No Calc |

Ring|  Ring|            1370 |          4840.85 |      1.73585 |      17.8679 |

Ring|   100|            1755 |          4841.66 |      3.11292 |      17.8679 |

100|  Ring|         0.99994 |          0.99994 |      No Calc |      No Calc |

Ring|  Ring|         1679.26 |          1679.26 |      1.67377 |      21.4269 |

Ring|   110|         0.99982 |          0.99982 |      0.52653 |      20.0339 |

--------------------------------------------------------------------------------

Factored Loads:

Un-Factored Loads:

Weights:

Fabricated - Bare  W/O Removable Internals                3201.3  kg.

Shop Test  - Fabricated + Water ( Full )                 44267.2  kg.

Shipping   - Fab. + Rem. Intls.+ Shipping App.            3201.3  kg.

Erected    - Fab. + Rem. Intls.+ Insul. (etc)             3201.3  kg.

Empty      - Fab. + Intls. + Details + Wghts.             3201.3  kg.

Operating  - Empty + Operating Liquid (No CA)             3201.3  kg.

Field Test - Empty Weight + Water (Full)                 44267.2  kg.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019