Table of Contents

Cover Sheet	2

Title Page	3

Warnings and Errors:	4

Input Echo:	5

XY Coordinate Calculations:	12

Internal Pressure Calculations:	13

External Pressure Calculations:	19

Element and Detail Weights:	29

Conical Section:	32

Conical Section:	37

Center of Gravity Calculation:	41

MDMT Summary:	42

Vessel Design Summary:	43

DESIGN CALCULATION

In Accordance with ASME Section VIII Division 1

ASME Code Version     : 2017

Analysis Performed by : SPLM Licensed User

Job File              : D:\2-PAYAM\1-PROJECTS\MICROTEC PROJECT\13-MICROT

Date of Analysis      : Feb 20,2026   4:19pm

PV Elite 2019 SP1, March 2019

Note:

PV Elite performs all calculations internally in Imperial Units

to remain compliant with the ASME Code and any built in assumptions

in the ASME Code formulas. The finalized results are reflected to show

the user's set of selected units.

Class From To : Basic Element Checks.

==========================================================================

Class From To: Check of Additional Element Data

==========================================================================

Please insure the C factor entered is in accordance with Figure UG-34.

Please insure the C factor entered is in accordance with Figure UG-34.

There were no geometry errors or warnings.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

PV Elite Vessel Analysis Program: Input Data

Design Internal Pressure (for Hydrotest)                 0.5  bars

Design Internal Temperature                            170.0  °C

Type of Hydrotest                              not Specified

Hydrotest Position                                  Vertical

Projection of Nozzle from Vessel Top                       0  mm.

Projection of Nozzle from Vessel Bottom                    0  mm.

Minimum Design Metal Temperature                       -28.9  °C

Type of Construction                                  Welded

Special Service                                         None

Degree of Radiography                                   RT-1

Use Higher Longitudinal Stresses (Flag)                    Y

Select t for Internal Pressure (Flag)                      N

Select t for External Pressure (Flag)                      N

Select t for Axial Stress (Flag)                           N

Select Location for Stiff. Rings (Flag)                    N

Consider Vortex Shedding                                   N

Perform a Corroded Hydrotest                               N

Load Case 1                                   NP+EW+WI+FW+BW

Load Case 2                                   NP+EW+EE+FS+BS

Load Case 3                                   NP+OW+WI+FW+BW

Load Case 4                                   NP+OW+EQ+FS+BS

Load Case 5                                         NP+HW+HI

Load Case 6                                         NP+HW+HE

Load Case 7                                   IP+OW+WI+FW+BW

Load Case 8                                   IP+OW+EQ+FS+BS

Load Case 9                                   EP+OW+WI+FW+BW

Load Case 10                                  EP+OW+EQ+FS+BS

Load Case 11                                        HP+HW+HI

Load Case 12                                        HP+HW+HE

Load Case 13                                        IP+WE+EW

Load Case 14                                        IP+WF+CW

Load Case 15                                        IP+VO+OW

Load Case 16                                        IP+VE+EW

Load Case 17                                        NP+VO+OW

Load Case 18                                     FS+BS+IP+OW

Load Case 19                                     FS+BS+EP+OW

Wind Design Code                                   ASCE-7 93

Basic Wind Speed                     [V]              112.65  Km/hr

Surface Roughness Category                   C: Open Terrain

Importance Factor                                        1.0

Type of Surface                            Moderately Smooth

Base Elevation                                             0  mm.

Percent Wind for Hydrotest                              33.0

Using User defined Wind Press. Vs Elev.                    N

Damping Factor (Beta) for Wind (Ope)                  0.0100

Damping Factor (Beta) for Wind (Empty)                0.0000

Damping Factor (Beta) for Wind (Filled)               0.0000

Seismic Design Code                                   UBC 94

UBC Seismic Zone (1=1,2=2a,3=2b,4=3,5=4)               0.000

UBC Importance Factor                                  1.000

UBC Soil Type                                          S1

UBC Horizontal Force Factor                            3.000

UBC Percent Seismic for Hydrotest                      0.000

Design Pressure + Static Head                              Y

Consider MAP New and Cold in Noz. Design                   N

Consider External Loads for Nozzle Des.                    Y

Use ASME VIII-1 Appendix 1-9                               N

Material Database Year        Current w/Addenda or Code Year

Configuration Directives:

Do not use Nozzle MDMT Interpretation VIII-1 01-37        No

Use Table G instead of exact equation for "A"            Yes

Shell Head Joints are Tapered                            Yes

Compute "K" in corroded condition                        Yes

Use Code Case 2286                                        No

Use the MAWP to compute the MDMT                         Yes

For thickness ratios <= 0.35, MDMT will be -155F (-104C) Yes

For PWHT & P1 Materials the MDMT can be < -55F (-48C)     No

Using Metric Material Databases, ASME II D                No

Calculate B31.3 type stress for Nozzles with Loads       Yes

Reduce the MDMT due to lower membrane stress             Yes

Consider Longitudinal Stress in MDMT calcs. (Div. 1)      No

Complete Listing of Vessel Elements and Details:

Element From Node                                         10

Element To Node                                           20

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                   50  mm.

Inside Diameter                                          321  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Allowable Stress, Ambient                          137.9  N./mm²

Allowable Stress, Operating                        136.1  N./mm²

Allowable Stress, Hydrotest                        179.3  N./mm²

Material Density                                0.008027  kg./cm³

P Number Thickness                                     0  mm.

Yield Stress, Operating                            156.1  N./mm²

External Pressure Chart Name                        HA-2

UNS Number                                        S31600

Product Form                                       Plate

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                       Ring 1-1

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                              327  mm.

Thickness of Ring                                     12  mm.

Outside Diameter of Ring                             405  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         20

Element To Node                                           30

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 1100  mm.

Inside Diameter                                          321  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1124  mm.

Design Length of Cone                                   1100  mm.

Half Apex Angle of Cone                            20.052082  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     20

Detail Type                                         Ring

Detail ID                                       Ring 2-1

Dist. from "FROM" Node / Offset dist                1095  mm.

Inside Diameter of Ring                           1126.3  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1227.3  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         30

Element To Node                                           40

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                  456  mm.

Inside Diameter                                         1124  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

--------------------------------------------------------------------

Element From Node                                         40

Element To Node                                           50

Element Type                                        Flat

Description

Distance "FROM" to "TO"                                   10  mm.

Inside Diameter                                         1124  mm.

Element Thickness                                         10  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                         10  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Flat Head Attachment Factor                              0.2

Small diameter if Non-Circular                             0  mm.

Weld is pre-Heated                                        No

--------------------------------------------------------------------

Element From Node                                         50

Element To Node                                           60

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                  115  mm.

Inside Diameter                                          566  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     50

Detail Type                                         Ring

Detail ID                                       Ring 5-1

Dist. from "FROM" Node / Offset dist                 108  mm.

Inside Diameter of Ring                              572  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                             670  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         60

Element To Node                                           70

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 3015  mm.

Inside Diameter                                          566  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1606  mm.

Design Length of Cone                                   3015  mm.

Half Apex Angle of Cone                            9.7855883  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     60

Detail Type                                         Ring

Detail ID                                       Ring 6-1

Dist. from "FROM" Node / Offset dist                   5  mm.

Inside Diameter of Ring                           571.73  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          669.73  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     60

Detail Type                                         Ring

Detail ID                                       Ring 6-2

Dist. from "FROM" Node / Offset dist                1874  mm.

Inside Diameter of Ring                           1218.4  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1314  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     60

Detail Type                                         Ring

Detail ID                                       Ring 6-3

Dist. from "FROM" Node / Offset dist                3010  mm.

Inside Diameter of Ring                           1610.3  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                          1705.9  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         70

Element To Node                                           80

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                 2514  mm.

Inside Diameter                                         1606  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     70

Detail Type                                         Ring

Detail ID                                       Ring 7-1

Dist. from "FROM" Node / Offset dist                 100  mm.

Inside Diameter of Ring                             1612  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1706  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

Element From Node                                     70

Detail Type                                         Ring

Detail ID                                       Ring 7-2

Dist. from "FROM" Node / Offset dist                1610  mm.

Inside Diameter of Ring                             1612  mm.

Thickness of Ring                                     10  mm.

Outside Diameter of Ring                            1706  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         80

Element To Node                                           90

Element Type                                        Flat

Description

Distance "FROM" to "TO"                                   14  mm.

Inside Diameter                                         1606  mm.

Element Thickness                                         14  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                         14  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Flat Head Attachment Factor                              0.2

Small diameter if Non-Circular                             0  mm.

Weld is pre-Heated                                        No

--------------------------------------------------------------------

Element From Node                                         90

Element To Node                                          100

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                  200  mm.

Inside Diameter                                          898  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     90

Detail Type                                         Ring

Detail ID                                       Ring 8.1

Dist. from "FROM" Node / Offset dist                 195  mm.

Inside Diameter of Ring                              904  mm.

Thickness of Ring                                     14  mm.

Outside Diameter of Ring                             956  mm.

Material Name                                 SA-240 316

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

XY Coordinate Calculations:

|    |            |            |             |             |

From     | To |  X (Horiz.)|  Y (Vert.) |  DX (Horiz.)|  DY (Vert.) |

|    |        mm. |        mm. |         mm. |         mm. |

----------------------------------------------------------------------

10     |  20|        ... |         50 |         ... |          50 |

20     |  30|        ... |       1150 |         ... |        1100 |

30     |  40|        ... |       1606 |         ... |         456 |

40     |  50|        ... |       1616 |         ... |          10 |

50     |  60|        ... |       1731 |         ... |         115 |

60     |  70|        ... |       4746 |         ... |        3015 |

70     |  80|        ... |       7260 |         ... |        2514 |

80     |  90|        ... |       7274 |         ... |          14 |

90     | 100|        ... |       7474 |         ... |         200 |

----------------------------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element Thickness, Pressure, Diameter and Allowable Stress :

|    | Int. Press |    Nominal | Total Corr |    Element |  Allowable |

From      | To | + Liq. Hd  |  Thickness |  Allowance |   Diameter |  Stress(SE)|

|    |       bars |        mm. |        mm. |        mm. |     N./mm² |

----------------------------------------------------------------------------------

10      |  20|        0.5 |          3 |        ... |        321 |      136.1 |

20      |  30|        0.5 |          3 |        ... |       1124 |      136.1 |

30      |  40|        0.5 |          3 |        ... |       1124 |     136.07 |

40      |  50|        0.5 |         10 |        ... |       1124 |     136.07 |

50      |  60|        0.5 |          3 |        ... |        566 |     136.07 |

60      |  70|        0.5 |          3 |        ... |       1606 |     136.07 |

70      |  80|        0.5 |          3 |        ... |       1606 |     136.07 |

80      |  90|        0.5 |         14 |        ... |       1606 |     136.07 |

90      | 100|        0.5 |          3 |        ... |        898 |     136.07 |

----------------------------------------------------------------------------------

Element Required Thickness and MAWP :

|    |     Design |   M.A.W.P. |     M.A.P. |    Minimum |   Required |

From      | To |   Pressure |   Corroded | New & Cold |  Thickness |  Thickness |

|    |       bars |       bars |       bars |        mm. |        mm. |

----------------------------------------------------------------------------------

10      |  20|        0.5 |    25.1557 |    25.4884 |          3 |        1.5 |

20      |  30|        0.5 |    6.80386 |    6.89384 |          3 |        1.5 |

30      |  40|        0.5 |    7.23977 |    7.33728 |          3 |        1.5 |

40      |  50|        0.5 |    2.12356 |    2.15217 |         10 |    4.85235 |

50      |  60|        0.5 |    14.3321 |    14.5251 |          3 |        1.5 |

60      |  70|        0.5 |    4.99817 |    5.06549 |          3 |        1.5 |

70      |  80|        0.5 |    5.07179 |    5.14011 |          3 |        1.5 |

80      |  90|        0.5 |    1.65349 |    1.67576 |         14 |    7.69861 |

90      | 100|        0.5 |    9.05452 |    9.17649 |          3 |        1.5 |

----------------------------------------------------------------------------------

Minimum                          1.653        1.676

MAWP: 1.653 bars, limited by: Flat Head.

Internal Pressure Calculation Results :

ASME Code, Section VIII Division 1, 2017

Cylindrical Shell From 10 To 20 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*160.5)/(136.1*1.0-0.6*0.5)

= 0.0590 + 0.0000 = 0.0590 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.1*1.0*3.0)/(160.5+0.6*3.0)

= 25.156 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(160.5+0.6*3.0)

= 25.488 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(160.5+0.6*3.0))/(1.0*3.0)

= 2.705 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.926 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 20 To 30 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1124.0)/(2*0.9394*(136.1*1.0-0.6*0.5))

= 0.2199 + 0.0000 = 0.2199 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*136.1*1.0*3.0*0.939)/(1124.0+1.2*3.0*0.939)

= 6.804 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9394)/(1124.0+1.2*3.0*0.9394)

= 6.894 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1124.0+1.2*3.0*0.9394))/(2*1.0*3.0*0.9394)

= 10.002 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.925 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

Note: The Pressure at the Large Diameter is used in the TR calculation.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 30 To 40 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*562.0)/(136.07*1.0-0.6*0.5)

= 0.2066 + 0.0000 = 0.2066 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.07*1.0*3.0)/(562.0+0.6*3.0)

= 7.240 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(562.0+0.6*3.0)

= 7.337 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(562.0+0.6*3.0))/(1.0*3.0)

= 9.397 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.266 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Welded Flat Head From 40 To 50 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= d * sqrt(Z*C*P/(S*E)) per UG-34 (c)(3)

= 566.0*sqrt(1.0*0.2*0.5/(136.07*1.0))

= 4.8524 + 0.0000 = 4.8524 mm.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (t/d)²*((S*E)/(C*Z)) UG-34 (c)(3)

= (10.0/566.0))²*((136.07*1.0)/(0.2*1.0))

= 2.124 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (t/d)²*((S*E)/(C*Z)) per UG-34 (c) (3)

= (10.0/566.0)²*((137.9*1.0)/(0.2*1.0))

= 2.152 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (Z*C*P)/(((t/d)²)*E)

= (1.0*0.2*0.5)/(((10.0/566.0)²)*1.0)

= 32.037 N./mm²

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 50 To 60 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*283.0)/(136.07*1.0-0.6*0.5)

= 0.1040 + 0.0000 = 0.1040 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.07*1.0*3.0)/(283.0+0.6*3.0)

= 14.332 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(283.0+0.6*3.0)

= 14.525 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(283.0+0.6*3.0))/(1.0*3.0)

= 4.747 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.527 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 60 To 70 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1606.0)/(2*0.9855*(136.07*1.0-0.6*0.5))

= 0.2995 + 0.0000 = 0.2995 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*136.07*1.0*3.0*0.985)/(1606.0+1.2*3.0*0.985)

= 4.998 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9855)/(1606.0+1.2*3.0*0.9855)

= 5.065 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1606.0+1.2*3.0*0.9855))/(2*1.0*3.0*0.9855)

= 13.612 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.527 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

Note: The Pressure at the Large Diameter is used in the TR calculation.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 70 To 80 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*803.0)/(136.07*1.0-0.6*0.5)

= 0.2952 + 0.0000 = 0.2952 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.07*1.0*3.0)/(803.0+0.6*3.0)

= 5.072 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(803.0+0.6*3.0)

= 5.140 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(803.0+0.6*3.0))/(1.0*3.0)

= 13.414 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.186 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Welded Flat Head From 80 To 90 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= d * sqrt(Z*C*P/(S*E)) per UG-34 (c)(3)

= 898.0*sqrt(1.0*0.2*0.5/(136.07*1.0))

= 7.6986 + 0.0000 = 7.6986 mm.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (t/d)²*((S*E)/(C*Z)) UG-34 (c)(3)

= (14.0/898.0))²*((136.07*1.0)/(0.2*1.0))

= 1.653 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (t/d)²*((S*E)/(C*Z)) per UG-34 (c) (3)

= (14.0/898.0)²*((137.9*1.0)/(0.2*1.0))

= 1.676 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (Z*C*P)/(((t/d)²)*E)

= (1.0*0.2*0.5)/(((14.0/898.0)²)*1.0)

= 41.145 N./mm²

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Cylindrical Shell From 90 To 100 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*449.0)/(136.07*1.0-0.6*0.5)

= 0.1650 + 0.0000 = 0.1650 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.07*1.0*3.0)/(449.0+0.6*3.0)

= 9.055 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(449.0+0.6*3.0)

= 9.176 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(449.0+0.6*3.0))/(1.0*3.0)

= 7.514 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.333 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Elements Suitable for Internal Pressure.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

External Pressure Calculation Results :

External Pressure Calculations:

|      |    Section |    Outside |  Corroded |    Factor  |     Factor |

From|   To |     Length |   Diameter | Thickness |       A    |        B   |

|      |        mm. |        mm. |       mm. |            |     N./mm² |

-----------------------------------------------------------------------------

10|  Ring|    No Calc |        ... |         3 |    No Calc |    No Calc |

Ring|    20|         50 |        327 |         3 |   0.010394 |    98.9477 |

20|  Ring|    706.449 |    1126.74 |         3 | 0.00026759 |    25.6883 |

Ring|    30|    4.99191 |    1130.39 |         3 |  0.0054666 |    92.8731 |

30|    40|        456 |       1130 |         3 | 0.00047288 |    45.3961 |

40|    50|    No Calc |        ... |        10 |    No Calc |    No Calc |

50|  Ring|        108 |        572 |         3 |  0.0031947 |    86.5975 |

Ring|    60|          7 |        572 |         3 |   0.021854 |    104.027 |

60|  Ring|    4.99249 |    573.813 |         3 |   0.021011 |    103.811 |

Ring|  Ring|    1374.53 |    1218.51 |         3 | 0.00014001 |    13.4413 |

Ring|  Ring|    997.779 |    1610.36 |         3 | 0.00017041 |    16.3596 |

Ring|    70|    4.99741 |    1612.09 |         3 |   0.003216 |    86.6818 |

70|  Ring|        100 |       1612 |         3 |  0.0024223 |    82.9383 |

Ring|  Ring|       1510 |       1612 |         3 | 0.00011383 |    10.9277 |

Ring|    80|        904 |       1612 |         3 |   0.000193 |     18.528 |

80|    90|    No Calc |        ... |        14 |    No Calc |    No Calc |

90|  Ring|        195 |        904 |         3 |   0.001308 |     73.751 |

Ring|   100|          5 |        904 |         3 |  0.0091683 |    97.8841 |

-----------------------------------------------------------------------------

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    20|          3 |    0.19212 |             0.1 |      12.103 |

20|  Ring|          3 |    1.27187 |             0.1 |     0.85662 |

Ring|    30|          3 |    0.43542 |             0.1 |     3.08702 |

30|    40|          3 |    0.99796 |             0.1 |     1.60685 |

40|    50|         10 |    No Calc |             0.1 |     No Calc |

50|  Ring|          3 |    0.36718 |             0.1 |     6.05541 |

Ring|    60|          3 |    0.20698 |             0.1 |     7.27422 |

60|  Ring|          3 |     0.2107 |             0.1 |     7.13086 |

Ring|  Ring|          3 |    1.66204 |             0.1 |     0.43479 |

Ring|  Ring|          3 |    1.72473 |             0.1 |     0.40042 |

Ring|    70|          3 |    0.59194 |             0.1 |     2.11938 |

70|  Ring|          3 |    0.64382 |             0.1 |      2.0579 |

Ring|  Ring|          3 |    2.02088 |             0.1 |     0.27114 |

Ring|    80|          3 |    1.63721 |             0.1 |     0.45972 |

80|    90|         14 |    No Calc |             0.1 |     No Calc |

90|  Ring|          3 |    0.61369 |             0.1 |     3.26313 |

Ring|   100|          3 |    0.32711 |             0.1 |      4.3309 |

-----------------------------------------------------------------------

Minimum                                                      0.271

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    20|              50 |          4811.25 |   0.00062658 |      9.67321 |

20|  Ring|         706.449 |          706.449 |      No Calc |      No Calc |

Ring|    30|         4.99191 |          4.99191 |      0.36588 |       20.652 |

30|    40|             456 |           6566.1 |      No Calc |      No Calc |

40|    50|         No Calc |          No Calc |      No Calc |      No Calc |

50|  Ring|             108 |          4021.26 |      No Calc |      No Calc |

Ring|    60|               7 |          487.092 |    0.0077134 |      17.0396 |

60|  Ring|         4.99249 |          4.99249 |      No Calc |      No Calc |

Ring|  Ring|         1374.53 |          1374.53 |     0.093369 |      17.0481 |

Ring|  Ring|         997.779 |          997.779 |      1.53939 |      18.2173 |

Ring|    70|         4.99741 |          4.99741 |       1.5006 |      19.1386 |

70|  Ring|             100 |          2398.86 |      No Calc |      No Calc |

Ring|  Ring|            1510 |          3984.95 |      2.40723 |      18.3065 |

Ring|    80|             904 |          3985.66 |      3.60856 |      18.3065 |

80|    90|         No Calc |          No Calc |      No Calc |      No Calc |

90|  Ring|             195 |          5498.92 |      No Calc |      No Calc |

Ring|   100|               5 |          216.544 |     0.052954 |      4.51783 |

--------------------------------------------------------------------------------

Elements Suitable for External Pressure.

ASME Code, Section VIII Division 1, 2017

Cone From 20 to Ring 2-1 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.818    1126.74      706.45    399.82    0.6270  0.0002676    25.69

EMAP = (4*B)/(3*(D/t)) = (4*25.6883)/(3*399.8153) = 0.8566 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.195    1126.74      706.45    943.05    0.6270  0.0000737     7.07

EMAP = (4*B)/(3*(D/t)) = (4*7.0738)/(3*943.0535) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 2-1 to 30 Ext. Chart: HA-2 at 50 °C

Cylindrical Shell From 30 to 40 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1130.00      456.00    376.67    0.4035  0.0004729    45.40

EMAP = (4*B)/(3*(D/t)) = (4*45.3961)/(3*376.6667) = 1.6068 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

0.998    1130.00      456.00   1132.32    0.4035  0.0000885     8.49

EMAP = (4*B)/(3*(D/t)) = (4*8.4934)/(3*1132.3154) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1130.00     6566.10    376.67    5.8107  0.0000295     2.83

EMAP = (4*B)/(3*(D/t)) = (4*2.8274)/(3*376.6667) = 0.1001 bars

Welded Flat Head

Note: This element's required thickness was computed in the internal

Pressure Report using the maximum of the Internal and External

pressures.

Cylindrical Shell From 50 to Ring 5-1 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000     572.00      108.00    190.67    0.1888  0.0031947    86.60

EMAP = (4*B)/(3*(D/t)) = (4*86.5975)/(3*190.6667) = 6.0554 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

0.367     572.00      108.00   1557.80    0.1888  0.0001217    11.68

EMAP = (4*B)/(3*(D/t)) = (4*11.6846)/(3*1557.8004) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000     572.00     4021.26    190.67    7.0302  0.0000649     6.23

EMAP = (4*B)/(3*(D/t)) = (4*6.2274)/(3*190.6667) = 0.4355 bars

Cone From 60 to Ring 6-1 Ext. Chart: HA-2 at 50 °C

Cone From Ring 6-1 to Ring 6-2 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.956    1218.51     1374.53    412.17    1.1280  0.0001400    13.44

EMAP = (4*B)/(3*(D/t)) = (4*13.4413)/(3*412.1667) = 0.4348 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.638    1218.51     1374.53    743.97    1.1280  0.0000581     5.58

EMAP = (4*B)/(3*(D/t)) = (4*5.5803)/(3*743.9673) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 6-2 to Ring 6-3 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.956    1610.36      997.78    544.71    0.6196  0.0001704    16.36

EMAP = (4*B)/(3*(D/t)) = (4*16.3596)/(3*544.7132) = 0.4004 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.700    1610.36      997.78    947.48    0.6196  0.0000740     7.11

EMAP = (4*B)/(3*(D/t)) = (4*7.1067)/(3*947.4755) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Ring 6-3 to 70 Ext. Chart: HA-2 at 50 °C

Cylindrical Shell From Ring 7-1 to Ring 7-2 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1612.00     1510.00    537.33    0.9367  0.0001138    10.93

EMAP = (4*B)/(3*(D/t)) = (4*10.9277)/(3*537.3333) = 0.2711 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.021    1612.00     1510.00    797.67    0.9367  0.0000623     5.98

EMAP = (4*B)/(3*(D/t)) = (4*5.983)/(3*797.673) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1612.00     3984.95    537.33    2.4721  0.0000420     4.03

EMAP = (4*B)/(3*(D/t)) = (4*4.0331)/(3*537.3333) = 0.1001 bars

Cylindrical Shell From Ring 7-2 to 80 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1612.00      904.00    537.33    0.5608  0.0001930    18.53

EMAP = (4*B)/(3*(D/t)) = (4*18.528)/(3*537.3333) = 0.4597 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.637    1612.00      904.00    984.60    0.5608  0.0000769     7.39

EMAP = (4*B)/(3*(D/t)) = (4*7.3851)/(3*984.6019) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000    1612.00     3985.66    537.33    2.4725  0.0000420     4.03

EMAP = (4*B)/(3*(D/t)) = (4*4.0324)/(3*537.3333) = 0.1001 bars

Welded Flat Head

Note: This element's required thickness was computed in the internal

Pressure Report using the maximum of the Internal and External

pressures.

Cylindrical Shell From 90 to Ring 8.1 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000     904.00      195.00    301.33    0.2157  0.0013080    73.75

EMAP = (4*B)/(3*(D/t)) = (4*73.751)/(3*301.3333) = 3.2631 bars

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

0.614     904.00      195.00   1473.05    0.2157  0.0001151    11.05

EMAP = (4*B)/(3*(D/t)) = (4*11.0491)/(3*1473.0518) = 0.1 bars

Results for Maximum Stiffened Length (Slen):

Tca         OD        SLEN       D/t       L/D   Factor A        B

3.000     904.00     5498.92    301.33    6.0829  0.0000385     3.70

EMAP = (4*B)/(3*(D/t)) = (4*3.696)/(3*301.3333) = 0.1635 bars

Stiffening Ring Calcs for : Ring 1-1 , SA-240 316 , Bar Ring: 39 x 12 mm.

Effective Length of Shell:                            34 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.034          1.5000           1.550

Ring :             4.680         22.5000         105.300

Total:             5.714                         106.850

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.008         17.2011         305.816

Ring :             5.932         -3.7989          67.540

Total:             5.940                         373.357

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         10 cm**4

Required Stress in Ring plus Shell      Breq        0.11 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000012

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (327.0²*25.0(3.0+4.68/25.0)0.000001)/10.9

= 0 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.03  Kgf/mm.

The Radial Shear Load                             V      0.08  Kgf

The First Moment of the Area ( Ring + Shell )     Q   1777.89  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.00  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     46.40  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.03  Kgf/mm.

Stiffening Ring Calcs for : Ring 2-1 , SA-240 316 , Bar Ring: 50 x 10 mm.

Effective Length of Shell:                            64 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.918          1.5000           2.877

Ring :             5.046         28.2300         142.449

Total:             6.964                         145.326

--------------------------------------------------------

Centroid of Ring plus Shell:                          21 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.014         19.3674         719.536

Ring :            10.707         -7.3627         273.537

Total:            10.721                         993.073

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         21 cm**4

Required Stress in Ring plus Shell      Breq        1.91 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000200

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1126.35²*355.7206(3.0+5.046/355.7206)0.00002)/10.9

= 0 cm**4

Stiffening Ring Calcs for : Ring 5-1 , SA-240 304 , Bar Ring: 49 x 10 mm.

Effective Length of Shell:                            46 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.367          1.5000           2.051

Ring :             4.900         27.5000         134.750

Total:             6.267                         136.801

--------------------------------------------------------

Centroid of Ring plus Shell:                          22 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.010         20.3287         564.925

Ring :             9.804         -5.6713         157.604

Total:             9.814                         722.529

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         17 cm**4

Required Stress in Ring plus Shell      Breq        0.37 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000039

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (572.0²*57.5(3.0+4.9/57.5)0.000004)/10.9

= 0 cm**4

Stiffening Ring Calcs for : Ring 6-1 , SA-240 316 , Bar Ring: 49 x 10 mm.

Effective Length of Shell:                            46 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.369          1.5000           2.054

Ring :             4.900         27.5000         134.750

Total:             6.269                         136.804

--------------------------------------------------------

Centroid of Ring plus Shell:                          22 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.010         20.3220         565.404

Ring :             9.804         -5.6780         157.975

Total:             9.814                         723.379

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         17 cm**4

Required Stress in Ring plus Shell      Breq        1.16 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000121

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (573.7247²*689.7628(3.0+4.9/689.7628)0.000012)/10.9

= 0 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                 INTERMITTENT

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.70  Kgf/mm.

The Radial Shear Load                             V      4.04  Kgf

The First Moment of the Area ( Ring + Shell )     Q   2782.23  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.07  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

Maximum Space between Welds  8*TCA                      50.80  mm.

The Weld Allowable Load                 WLeg*0.55*S     23.20  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.71  Kgf/mm.

Stiffening Ring Calcs for : Ring 6-2 , SA-240 316 , Bar Ring: 48 x 10 mm.

Effective Length of Shell:                            67 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.995          1.5000           2.993

Ring :             4.782         26.9100         128.684

Total:             6.777                         131.676

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.015         17.9295         641.370

Ring :             9.113         -7.4805         267.592

Total:             9.128                         908.962

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        2.69 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000280

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1218.4214²*1186.1562(3.0+4.782/1186.1562)0.000028)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                 INTERMITTENT

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.21  Kgf/mm.

The Radial Shear Load                             V     14.74  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3577.18  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.29  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

Maximum Space between Welds  8*TCA                      50.80  mm.

The Weld Allowable Load                 WLeg*0.55*S     23.20  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.24  Kgf/mm.

Stiffening Ring Calcs for : Ring 6-3 , SA-240 316 , Bar Ring: 48 x 10 mm.

Effective Length of Shell:                            76 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.294          1.5000           3.440

Ring :             4.782         26.9100         128.684

Total:             7.076                         132.124

--------------------------------------------------------

Centroid of Ring plus Shell:                          19 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.017         17.1731         676.429

Ring :             9.113         -8.2369         324.442

Total:             9.130                        1000.870

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         19 cm**4

Required Stress in Ring plus Shell      Breq        3.05 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000318

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1610.2754²*501.3885(3.0+4.782/501.3885)0.000032)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                 INTERMITTENT

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.51  Kgf/mm.

The Radial Shear Load                             V      8.23  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3938.88  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.17  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

Maximum Space between Welds  8*TCA                      50.80  mm.

The Weld Allowable Load                 WLeg*0.55*S     23.20  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.54  Kgf/mm.

Stiffening Ring Calcs for : Ring 7-1 , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            76 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.295          1.5000           3.442

Ring :             4.700         26.5000         124.550

Total:             6.995                         127.993

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.017         16.7981         647.554

Ring :             8.652         -8.2020         316.180

Total:             8.669                         963.734

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.37 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000350

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1612.0²*805.0(3.0+4.7/805.0)0.000035)/10.9

= 2 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      0.82  Kgf/mm.

The Radial Shear Load                             V     13.23  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3854.93  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.28  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     46.40  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      0.87  Kgf/mm.

Stiffening Ring Calcs for : Ring 7-2 , SA-240 316 , Bar Ring: 47 x 10 mm.

Effective Length of Shell:                            76 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.295          1.5000           3.442

Ring :             4.700         26.5000         124.550

Total:             6.995                         127.993

--------------------------------------------------------

Centroid of Ring plus Shell:                          18 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.017         16.7981         647.554

Ring :             8.652         -8.2020         316.180

Total:             8.669                         963.734

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         18 cm**4

Required Stress in Ring plus Shell      Breq        3.57 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000370

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1612.0²*1207.0(3.0+4.7/1207.0)0.000037)/10.9

= 4 cm**4

Results for Stiffening Ring Weld Calculations per UG-30:

Given Stiffening Ring Fillet Weld Size         Wleg     3.000  mm.

Stiffening Ring Attachment Style                   CONTINUOUS

Location of Stiffening Ring                          EXTERNAL

Radial Pressure Load                      Pext*Slen      1.23  Kgf/mm.

The Radial Shear Load                             V     19.84  Kgf

The First Moment of the Area ( Ring + Shell )     Q   3854.93  mm.³

Weld Shear Flow due to Rad. Shear Load         VQ/I      0.42  Kgf/mm.

The Weld Allowable Stress                    0.55*S     75.85  N./mm²

Minimum Weld Leg Size Min( 6mm, t, tw )      Wldmin      3.00  mm.

The Weld Allowable Load            2*   WLeg*0.55*S     46.40  Kgf/mm.

The Combined Weld Load   SRSS of VQ/I and Pext*Slen      1.30  Kgf/mm.

Stiffening Ring Calcs for : Ring 8.1 , SA-240 316 , Bar Ring: 26 x 14 mm.

Effective Length of Shell:                            57 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.719          1.5000           2.578

Ring :             3.640         16.0000          58.240

Total:             5.359                          60.818

--------------------------------------------------------

Centroid of Ring plus Shell:                          11 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.013          9.8497         166.726

Ring :             2.051         -4.6503          78.716

Total:             2.063                         245.442

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:          5 cm**4

Required Stress in Ring plus Shell      Breq        1.02 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000106

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (904.0²*100.0(3.0+3.64/100.0)0.000011)/10.9

= 0 cm**4

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element and Detail Weights:

|    |    Element |    Element |  Corroded |   Corroded | Extra due  |

From| To | Metal Wgt. | ID  Volume |Metal Wgt. |  ID Volume | Misc %     |

|    |        kg. |        Cm. |       kg. |        Cm. |        kg. |

---------------------------------------------------------------------------

10|  20|    1.22562 |    4047.14 |   1.22562 |    4047.14 |        ... |

20|  30|    64.2908 |     497493 |   64.2908 |     497493 |        ... |

30|  40|    38.8802 |     452548 |   38.8802 |     452548 |        ... |

40|  50|    79.6512 |        ... |   79.6512 |        ... |        ... |

50|  60|    4.95053 |      28940 |   4.95053 |      28940 |        ... |

60|  70|    252.081 |    3006753 |   252.081 |    3006753 |        ... |

70|  80|    306.025 |    5093591 |   306.025 |    5093591 |        ... |

80|  90|    227.656 |        ... |   227.656 |        ... |        ... |

90| 100|    13.6332 |     126692 |   13.6332 |     126692 |        ... |

---------------------------------------------------------------------------

Total |        988 | 9210065.00 |       988 | 9210065.00 |          0 |

Weight of Details:

|    |  Weight of |  X Offset, | Y Offset, |

From|Type|     Detail | Dtl. Cent. |Dtl. Cent. |   Description

|    |        kg. |        mm. |       mm. |

-------------------------------------------------

10|Ring|    4.31963 |        ... |       ... |   Ring 1-1

20|Ring|    14.9752 |        ... |      1095 |   Ring 2-1

50|Ring|    7.67375 |        ... |       108 |   Ring 5-1

60|Ring|    7.67041 |        ... |         5 |   Ring 6-1

60|Ring|    15.2695 |        ... |      1874 |   Ring 6-2

60|Ring|    19.9959 |        ... |      3010 |   Ring 6-3

70|Ring|    19.6637 |        ... |       100 |   Ring 7-1

70|Ring|    19.6637 |        ... |      1610 |   Ring 7-2

90|Ring|    8.53696 |        ... |       195 |   Ring 8.1

-------------------------------------------------

Total Weight of Each Detail Type:

Stiffeners                        117.8

-------------------------------------------

Sum of the Detail Weights         117.8 kg.

Weight Summation Results: (kg.)

| Fabricated |  Shop Test |   Shipping |    Erected |      Empty |  Operating |

---------------------------------------------------------------------------------------------

Main Elements |      988.4 |      988.4 |      988.4 |      988.4 |      988.4 |      988.4 |

Stif. Rings |      117.8 |      117.8 |      117.8 |      117.8 |      117.8 |      117.8 |

Test Liquid |        ... |     9204.4 |        ... |        ... |        ... |        ... |

---------------------------------------------------------------------------------------------

Totals |     1106.2 |    10310.6 |     1106.2 |     1106.2 |     1106.2 |     1106.2 |

Weight Summary:

Fabricated Wt.  - Bare Weight without Removable Internals                  1106.2 kg.

Shop Test Wt.   - Fabricated Weight + Water ( Full )                      10310.6 kg.

Shipping Wt.    - Fab. Weight + removable Intls.+ Shipping App.            1106.2 kg.

Erected  Wt.    - Fab. Wt + or - loose items (trays,platforms etc.)        1106.2 kg.

Ope. Wt. no Liq - Fab. Weight + Internals. + Details + Weights             1106.2 kg.

Operating Wt.   - Empty Weight + Operating Liq. Uncorroded                 1106.2 kg.

Field Test Wt.  - Empty Weight + Water (Full)                             10310.6 kg.

Mass of the Upper 1/3 of the Vertical Vessel                                546.7 kg.

Outside Surface Areas of Elements:

|    |        Surface |

From| To |           Area |

|    |            cm² |

----------------------------

10|  20|         513.65 |

20|  30|          26814 |

30|  40|          16188 |

40|  50|            ... |

50|  60|        2066.54 |

60|  70|         104969 |

70|  80|         127315 |

80|  90|            ... |

90| 100|           5680 |

----------------------------

Total        283546.125 cm²

Element and Detail Weights:

| To |  Total Ele.| Total. Ele.|Total. Ele.|  Total Dtl.| Oper. Wgt. |

From| To |  Empty Wgt.|  Oper. Wgt.|Hydro. Wgt.| Offset Mom.| No Liquid  |

|    |        kg. |        kg. |       kg. |      Kg-m. |        kg. |

---------------------------------------------------------------------------

10|  20|    5.54525 |    5.54525 |   5.54525 |        ... |    5.54525 |

20|  30|    79.2661 |    79.2661 |   79.2661 |        ... |    79.2661 |

30|  40|    38.8802 |    38.8802 |   38.8802 |        ... |    38.8802 |

40|  50|    79.6512 |    79.6512 |   79.6512 |        ... |    79.6512 |

50|  60|    12.6243 |    12.6243 |   12.6243 |        ... |    12.6243 |

60|  70|    295.017 |    295.017 |   295.017 |        ... |    295.017 |

70|  80|    345.352 |    345.352 |   345.352 |        ... |    345.352 |

80|  90|    227.656 |    227.656 |   227.656 |        ... |    227.656 |

90| 100|    22.1701 |    22.1701 |   22.1701 |        ... |    22.1701 |

---------------------------------------------------------------------------

Cumulative Vessel Weight

|    | Cumulative Ope | Cumulative | Cumulative  |

From| To | Wgt. No Liquid | Oper. Wgt. | Hydro. Wgt. |

|    |            kg. |        kg. |         kg. |

-------------------------------------------------------

10|  20|        1106.16 |    1106.16 |     1106.16 |

20|  30|        1100.62 |    1100.62 |     1100.62 |

30|  40|        1021.35 |    1021.35 |     1021.35 |

40|  50|        982.471 |    982.471 |     982.471 |

50|  60|         902.82 |     902.82 |      902.82 |

60|  70|        890.195 |    890.195 |     890.195 |

70|  80|        595.178 |    595.178 |     595.178 |

80|  90|        249.826 |    249.826 |     249.826 |

90| 100|        22.1701 |    22.1701 |     22.1701 |

-------------------------------------------------------

Note: The cumulative operating weights no liquid in the column above

are the cumulative operating weights minus the operating liquid

weight minus any weights absent in the empty condition.

Cumulative Vessel Moment

|    | Cumulative | Cumulative |Cumulative |

From| To | Empty Mom. | Oper. Mom. |Hydro. Mom.|

|    |      Kg-m. |      Kg-m. |     Kg-m. |

-------------------------------------------------

10|  20|        ... |        ... |       ... |

20|  30|        ... |        ... |       ... |

30|  40|        ... |        ... |       ... |

40|  50|        ... |        ... |       ... |

50|  60|        ... |        ... |       ... |

60|  70|        ... |        ... |       ... |

70|  80|        ... |        ... |       ... |

80|  90|        ... |        ... |       ... |

90| 100|        ... |        ... |       ... |

-------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 20 To 30 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    10.2926  mm.

Maximum Centroid Reinforcement Distance Small End     5.5368  mm.

Note: No ring was found close enough to the small end to be considered.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00037

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                      20.052  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Erl ), 1 )

= max( 0.25122E+11/( 136.1 * 0.18458E+09 ), 1 )

= 1.0002

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 136.1 * 0.18458E+09

= 25121794048.0 N./mm²^2

Decay Length, Cone Large End:

= 2 * sqrt( RL( ts - ca ) )

= 2 * sqrt( 562.0( 3.0 - 0.0 ) )

= 82.122 mm.

Required Area of Reinforcement, Large End, Internal [Arl]:

= k*QL*Rl/(Ss*E1))*(1-delta/alpha)*tan(alpha)

= 1.0 * 1.4328 * 562.0/( 136 * 1.0 ) *

( 1 - 11.0/20.05 ) * 0.365

= 0.0956 cm²

Force per Length, Cone Large End [QL]:

= P( Rl/2 ) - Faxial/(pi(Dl + Tl)) + Moment/(pi(Rl + Tl/2)(Rl + Tl/2))

= 0.5( 562.0/2 ) - 0.0/(pi(1124.0 + 3.0)) +

0.0/(pi(562.0 + 3.0/2)(562.0 + 3.0/2))

= 1.433 Kgf/mm.

Area of Reinforcement Available in Large end Shell [Ael]:

= (Ts-t)*sqrt(Rl*Ts)+(Tc-Tr)*sqrt(Rl*Tc/cos(alpha))

= ( 3.0 - 0.2066 ) * SQRT( 562.0 * 3.0 ) +

( 3.0 - 0.2199 ) * SQRT( 562.0 * 3.0/0.9394 )

= 2.3248 cm²

Summary of Reinforcement area, Large end, Internal Pressure:

Area of reinforcement required per App. 1-5(1)        0.0956  cm²

Area of reinforcement in shell per App. 1-5(2)        2.3248  cm²

Area of reinforcement in stiffening ring              5.0460  cm²

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.181  degrees

Large end actual half apex angle                      20.052  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.26602E+11/( 137.9 * 0.19291E+09 ), 1 )

= 1.0000

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 137.9 * 0.19291E+09

= 26602469376.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   137.9  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        137.9  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0*0.2881*565.0*0.365/(138*1.0))*

(1-1/4((0.1*565.0-0.288)/0.288)*(0.181/20.052)

= 0.0422 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 565.0/2 ) + 0.0/(pi(1130.0 - 3.0)) +

0.0/(pi(565.0 - 3.0/2)(565.0 - 3.0/2))

= 0.288 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1130.0 * 3.0 )½ * ( 3.0 + 3.0/0.939 )

= 1.9834 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0422  cm²

Area of reinforcement in shell per App. 1-8(2)        1.9834  cm²

Area of reinforcement in stiffening ring              5.0460  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       29.45  cm²

Force per Unit Length on Shell / Cone Junction          0.61  Kgf/mm.

Actual Buckling Stress associated with this Force       1.72  N./mm²

Material Strain associated with this stress         0.000018

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.178E-04 * 1130.0 * 1130.0 * 29.45/10.9

= 6151.72 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      0.961    0.0000     0.000        18.3378     0.007   323.055

Con      1.023   -5.8442    -5.977        24.1820     0.125   598.033

Sec      5.046   26.7300   134.880        -8.3923    10.707   355.391

TOT      7.029             128.903                   10.839  1276.479

Centroid of Section        18.3378     Moment of Inertia        23.60

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              23.604  cm**4

Required  Moment of Inertia ( Large End )               0.615  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00037

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                      20.052  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.25122E+11/( 136.1 * 0.18458E+09 ), 1 )

= 1.0002

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 136.1 * 0.18458E+09

= 25121794048.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 160.5( 3.0 - 0.0 ) )

= 30.720 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.0 * 0.4092 * 160.5/( 136 * 1.0 ) *

( 1 - 4.0/20.05 ) 0.365

= 0.0138 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 160.5/2 ) - 0.0/(pi(321.0 + 3.0)) +

0.0/(pi(160.5 + 3.0/2)(160.5 + 3.0/2))

= 0.409 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(160.5*3.0)½*(( 3.0- 0.059 )+( 3.0- 0.063 )/0.94)

= 1.0385 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0138  cm²

Area of reinforcement in shell per App. 1-5(4)        1.0385  cm²

Area of reinforcement in stiffening ring              0.0000  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    137.9  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         137.9  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.26602E+11/( 137.9 * 0.19291E+09 ), 1 )

= 1.0000

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 137.9 * 0.19291E+09

= 26602469376.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0*0.0834*163.5*0.365/(138*1.0))

= 0.00354 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 163.5/2 ) + 0.0/(pi(327.0 - 3.0)) +

0.0/(pi(163.5 - 3.0/2)(163.5 - 3.0/2))

= 0.083 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(327.0*3.0)½*[(3.0-1.272)+(3.0-0.205)/0.939]

= 0.8103 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0035  cm²

Area of reinforcement in shell per App. 1-8(4)        0.8103  cm²

Area of reinforcement in stiffening ring              0.0000  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       18.31  cm²

Force per Unit Length on Shell / Cone Junction          0.89  Kgf/mm.

Actual Buckling Stress associated with this Force       1.17  N./mm²

Material Strain associated with this stress         0.000012

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.121E-04 * 327.0 * 327.0 * 18.31/10.9

= 217.43 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      0.517    0.0000     0.000         1.6211     0.004     1.358

Con      0.550    3.1438     1.730        -1.5228     0.023     1.276

Sec      0.000    1.5000     0.000         0.1211     0.000     0.000

TOT      1.067               1.730                    0.027     2.634

Centroid of Section         1.6211     Moment of Inertia         0.05

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.5301E-01  cm**4

Required  Moment of Inertia ( Small End )            0.2174E-01  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress            5.51          408.30  Small Cyl. Long.

Compres. Stress           -2.81         -408.30  Small Cyl. Long.

Membrane Stress            4.99         -204.15   Small End Tang.

Tensile  Stress            5.60          408.30 Cone Longitudinal

Compres. Stress           -2.72         -408.30 Cone Longitudinal

Compres  Stress            5.16         -204.15   Cone Tangential

Tensile  Stress           31.69          408.30  Large Cyl. Long.

Compres. Stress          -22.30         -408.30  Large Cyl. Long.

Membrane Stress           -5.47         -204.15   Large End Tang.

Tensile  Stress           31.99          408.30 Cone Longitudinal

Compres. Stress          -21.99         -408.30 Cone Longitudinal

Compres  Stress           -4.86         -204.15   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         6.804     Thickness due to internal pressure, Cone Large End

MAPnc         6.893     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Conical Reinforcement Calculations, ASME VIII Div. 1, App. 1

Conical Section From 60 To 70 SA-240 316

Elastic Modulus Data from ASME Section II Part D at 50 °C

Elastic Modulus of Cone Material                 0.193E+09 KPa. at 50 °C

Elastic Modulus of Small Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large Cylinder Material       0.193E+09 KPa. at 50 °C

Elastic Modulus of Large End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus of Small End Reinforcement       0.193E+09 KPa. at 50 °C

Elastic Modulus Data from ASME Section II Part D at 170 °C

Elastic Modulus of Cone Material                 0.185E+09 KPa. at 170 °C

Elastic Modulus of Small Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large Cylinder Material       0.185E+09 KPa. at 170 °C

Elastic Modulus of Large End Reinforcement       0.185E+09 KPa. at 170 °C

Elastic Modulus of Small End Reinforcement       0.185E+09 KPa. at 170 °C

Axial Force on Small End of Cone                        0.00  Kgf

Axial Force on Large End of Cone                        0.00  Kgf

Moment on Small End of Cone                             0.00  Kg-m.

Moment on Large End of Cone                             0.00  Kg-m.

Note:  Since there are no supports, the forces and moments could

not be resolved to the large and small ends of the cone.

Note: Both ends of the Cone are Lines of Support

Maximum Centroid Reinforcement Distance Large End    12.2933  mm.

Maximum Centroid Reinforcement Distance Small End     7.3229  mm.

Reinforcement Calculations for Cone / Large Cylinder:

Required Area of Reinforcement for Large End Under Internal Pressure

Large end ratio of pressure to allowable stress      0.00037

Large end max. half apex angle w/o reinforcement      11.000  degrees

Large end actual half apex angle                       9.786  degrees

Required Area of Reinforcement for Large End Under External Pressure

Large end ratio of pressure to allowable stress      0.00007

Large end max. half apex angle w/o reinforcement       0.181  degrees

Large end actual half apex angle                       9.786  degrees

Intermediate Value [k]:

= max( Y / ( Srl * Erl ), 1 )

= max( 0.26602E+11/( 137.9 * 0.19291E+09 ), 1 )

= 1.0000

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 137.9 * 0.19291E+09

= 26602469376.0 N./mm²^2

Allowable Stress of Large End Material (Ext. Temp)   137.9  N./mm²

Allowable Stress of Cone Material (Ext. Temp)        137.9  N./mm²

Required Area of Reinforcement, Large End, External [Arl]:

= (k*QL*Rl*tan(angle)/(Ss*E1))*(1-1/4((P*Rl-QL)/QL)*(delta/alpha)

= (1.0*0.411*806.0*0.172/(138*1.0))*

(1-1/4((0.1*806.0-0.411)/0.411)*(0.181/9.786)

= 0.0404 cm²

Force per Length, Cone Large End External Pressure [QL]:

= Pext( Rl/2 ) + Faxial/(pi(Dl - Tl)) + Moment/(pi(Rl - Tl/2)(Rl - Tl/2))

= 0.1( 806.0/2 ) + 0.0/(pi(1612.0 - 3.0)) +

0.0/(pi(806.0 - 3.0/2)(806.0 - 3.0/2))

= 0.411 Kgf/mm.

Available Area of Reinforcement, Large End, External [Ael]:

= 0.55*( Dl*ts )½ * ( ts + tc/cos(alpha) )

= 0.55 * ( 1612.0 * 3.0 )½ * ( 3.0 + 3.0/0.985 )

= 2.3118 cm²

Summary of Reinforcement Area, Large End, External Pressure:

Area of reinforcement required per App. 1-8(1)        0.0404  cm²

Area of reinforcement in shell per App. 1-8(2)        2.3118  cm²

Area of reinforcement in stiffening ring              4.7820  cm²

Intermediate Results, Large End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       52.17  cm²

Force per Unit Length on Shell / Cone Junction          1.37  Kgf/mm.

Actual Buckling Stress associated with this Force       3.11  N./mm²

Material Strain associated with this stress         0.000032

Required Moment of Inertia, Large End, External Pressure [I's]:

= A * Dl² * Atl / 10.9

= 0.322E-04 * 1612.0 * 1612.0 * 52.17/10.9

= 40104.17 mm.4

Available Moment of Inertia, Large End, External Pressure:

Area  Centroid     Ar*Ce           Dist        I    Ar*Di²

Shl      1.147    0.0000     0.000        16.5877     0.009   315.719

Con      1.164   -3.2983    -3.840        19.8860     0.051   460.457

Sec      4.782   25.4100   121.511        -8.8223     9.113   372.194

TOT      7.094             117.670                    9.173  1148.371

Centroid of Section        16.5877     Moment of Inertia        20.66

Summary of Large End Inertia Calculations

Available Moment of Inertia ( Large End )              20.656  cm**4

Required  Moment of Inertia ( Large End )               4.010  cm**4

Reinforcement Calculations for Cone / Small Cylinder:

Required Area of Reinforcement for Small End under Internal Pressure

Small end ratio of pressure to allowable stress      0.00037

Small end max. half apex angle w/o reinforcement       4.000  degrees

Small end actual half apex angle                       9.786  degrees

Intermediate Value [k]:

= max( Y / ( Sr * Ers ), 1 )

= max( 0.25116E+11/( 136.1 * 0.18458E+09 ), 1 )

= 1.0000

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Int. temp.)

= 136.1 * 0.18458E+09

= 25115742208.0 N./mm²^2

Decay Length, Cone Small End:

= 1.4 * sqrt( Rs( ts - ca ) )

= 1.4 * sqrt( 283.0( 3.0 - 0.0 ) )

= 40.793 mm.

Required Area of Reinforcement, Small End, Internal [Ars]:

= k*QS*Rs/(Ss*E1)*(1-delta/alpha)*tan(alpha)

= 1.0 * 0.7215 * 283.0/( 136 * 1.0 ) *

( 1 - 4.0/9.79 ) 0.1725

= 0.015 cm²

Force per Length, Cone Small End [QS]:

= P( Rs/2 ) - Faxial/(pi(Ds + Ts)) + Moment/(pi(Rs + Ts/2)(Rs + Ts/2))

= 0.5( 283.0/2 ) - 0.0/(pi(566.0 + 3.0)) +

0.0/(pi(283.0 + 3.0/2)(283.0 + 3.0/2))

= 0.721 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.78(Rs*Ts)½*((Ts-t)+(Tc-Tr)/cos(alpha) ))

= 0.78(283.0*3.0)½*(( 3.0- 0.104 )+( 3.0- 0.106 )/0.99)

= 1.3257 cm²

Summary of Reinforcement Area, Small End, Internal Pressure:

Area of reinforcement required per App. 1-5(3)        0.0150  cm²

Area of reinforcement in shell per App. 1-5(4)        1.3257  cm²

Area of reinforcement in stiffening ring              4.9000  cm²

Required Area of Reinforcement for Small End Under External Pressure

Allowable Stress of Small End Material (Ext. Temp)    137.9  N./mm²

Allowable Stress of Cone Material (Ext. Temp)         137.9  N./mm²

Intermediate Value [k]:

= max( Y / ( Srs * Ers ), 1 )

= max( 0.26602E+11/( 137.9 * 0.19291E+09 ), 1 )

= 1.0000

where [Y] is:

= Cone Allowable Stress * Cone Elastic Modulus (Ext. temp.)

= 137.9 * 0.19291E+09

= 26602469376.0 N./mm²^2

Area of Reinforcement Required in Small End Shell [Ars]:

= k * QS * Rs * tan(alpha) / ( Ss * E1 )

= (1.0*0.1458*286.0*0.1725/(138*1.0))

= 0.00512 cm²

Force per Length, Cone Small End [QS]:

= Pext( Rs/2 ) + Faxial/(pi(Ds - Ts)) + Moment/(pi(Rs - Ts/2)(Rs - Ts/2))

= 0.1( 286.0/2 ) + 0.0/(pi(572.0 - 3.0)) +

0.0/(pi(286.0 - 3.0/2)(286.0 - 3.0/2))

= 0.146 Kgf/mm.

Area of Reinforcement Available in Small End Shell [Aes]:

= 0.55*(Ds*ts)½*[(ts-t)+(tc-tr)/cos(alpha))]

= 0.55*(572.0*3.0)½*[(3.0-0.211)+(3.0-0.21)/0.985]

= 1.2805 cm²

Summary of Reinforcement Area, Small End, External Pressure:

Area of reinforcement required per App. 1-8(3)        0.0051  cm²

Area of reinforcement in shell per App. 1-8(4)        1.2805  cm²

Area of reinforcement in stiffening ring              4.9000  cm²

Intermediate Results, Small End, External Pressure:

Area Available in Cone, Shell, and Reinforcement       52.41  cm²

Force per Unit Length on Shell / Cone Junction          2.04  Kgf/mm.

Actual Buckling Stress associated with this Force       1.63  N./mm²

Material Strain associated with this stress         0.000017

Required Moment of Inertia, Small End, External Pressure [I's]:

= A * Ds² * Ats / 10.9

= 0.169E-04 * 572.0 * 572.0 * 52.41/10.9

= 2666.48 mm.4

Available Moment of Inertia, Small End, External Pressure:

Area  Centroid     Ar*Ce           Dist         I    Ar*Di²

Shl      0.684    0.0000     0.000        20.5131     0.005   287.610

Con      0.694    1.9648     1.363        18.5483     0.014   238.626

Sec      4.900   26.0000   127.400        -5.4869     9.804   147.521

TOT      6.277             128.763                    9.823   673.757

Centroid of Section        20.5131     Moment of Inertia        16.56

Summary of Small End Inertia Calculations

Available Moment of Inertia ( Small End )            0.1656E+02  cm**4

Required  Moment of Inertia ( Small End )            0.2666E+00  cm**4

Note: The following calculations are only required per 1-5(g)(1) and

do include external loads due to wind or seismic.  These

discontinuity stresses are computed at the shell/cone junction

and do not include effects of local stiffening from a junction ring.

Results for Discontinuity Stresses per Bednar p. 236 2nd Edition

Stress Type              Stress      Allowable          Location

Tensile  Stress            7.00          408.20  Small Cyl. Long.

Compres. Stress           -2.26         -408.20  Small Cyl. Long.

Membrane Stress            7.29         -204.10   Small End Tang.

Tensile  Stress            7.04          408.20 Cone Longitudinal

Compres. Stress           -2.23         -408.20 Cone Longitudinal

Compres  Stress            7.36         -204.10   Cone Tangential

Tensile  Stress           28.73          408.20  Large Cyl. Long.

Compres. Stress          -15.32         -408.20  Large Cyl. Long.

Membrane Stress            1.29         -204.10   Large End Tang.

Tensile  Stress           28.83          408.20 Cone Longitudinal

Compres. Stress          -15.22         -408.20 Cone Longitudinal

Compres  Stress            1.49         -204.10   Cone Tangential

Note: An asterisk (*) denotes that this stress was not applicable

for this combination of loads.

Maximum Allowable Pressure Calculations for Cone to Shell Junction:

Pressure      Pressure                                                 Reason

Case          bars                           for Failure at this Pressure

-------------------------------------------------------------------------------

MAWP         4.998     Thickness due to internal pressure, Cone Large End

MAPnc         5.065     Thickness due to internal pressure, Cone Large End

These pressures were determined by iteration.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Shop/Field Installation Options :

Note : The CG is computed from the first Element From Node

Center of Gravity of Stiffening Rings                      4055.186 mm.

Center of Gravity of Bare Shell New and Cold               4759.567 mm.

Center of Gravity of Bare Shell Corroded                   4759.567 mm.

Vessel CG in the Operating Condition                       4684.575 mm.

Vessel CG in the Fabricated (Shop/Empty) Condition         4684.575 mm.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Minimum Design Metal Temperature Results Summary :

Curve  Basic  Reduced  UG-20(f)  Thickness     Gov      E*   PWHT

Description               MDMT     MDMT      MDMT      ratio     Thk           reqd

Notes           °C       °C        °C                mm.

-----------------------------------------------------------------------------------

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

[16]         -196

-----------------------------------------------------------------------------------

Warmest MDMT:             -196

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Notes:

[  ! ] - This was an impact tested material.

[  1] - Governing Nozzle Weld.

[  4] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-c).

[  5] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-b).

[  6] - MDMT Calculations at the Shell/Head Joint.

[  7] - MDMT Calculations for the Straight Flange.

[  8] - Cylinder/Cone/Flange Junction MDMT.

[  9] - Calculations in the Spherical Portion of the Head.

[10] - Calculations in the Knuckle Portion of the Head.

[11] - Calculated (Body Flange) Flange MDMT.

[12] - Calculated Flat Head MDMT per UCS-66.3

[13] - Tubesheet MDMT, shell side, if applicable

[14] - Tubesheet MDMT, tube side, if applicable

[15] - Nozzle Material

[16] - Shell or Head Material

[17] - Impact Testing required

[18] - Impact Testing not required, see UCS-66(b)(3)

[19] - Select a valid hydrotest type to get the UG-20(f) exemption

[20] - Cylinder/Cone Junction MDMT based on Longitudinal Stress considerations

[21] - Bolting Material

UG-84(b)(2) was not considered.

UCS-66(g) was not considered.

UCS-66(i) was not considered.

Notes:

Impact test temps were not entered in and not considered in the analysis.

UCS-66(i) applies to impact tested materials not by specification and

UCS-66(g) applies to materials impact tested per UG-84.1 General Note (c).

The Basic MDMT includes the (30F) PWHT credit if applicable.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

ASME Code, Section VIII Division 1, 2017

Diameter Spec : 321.000 x 1124.000 x 566.000 x 1606.000 x 898.000 mm. ID

Vessel Design Length, Tangent to Tangent           7474.00  mm.

Distance of Bottom Tangent above Grade               50.00  mm.

Distance of Base above Grade                          0.00  mm.

Specified Datum Line Distance                         0.00  mm.

Internal Design Temperature                            170  °C

Internal Design Pressure                             0.500  bars

External Design Temperature                             50  °C

External Design Pressure                             0.100  bars

Maximum Allowable Working Pressure                   1.653  bars

External Max. Allowable Working Pressure             0.271  bars

Hydrostatic Test Pressure                            0.000  bars

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Wind Design Code                                   ASCE-93

Earthquake Design Code                              UBC-94

Materials of Construction:

Component |                 |       |                        |         |Normal |  Impact |

Type |        Material | Class |              Thickness |   UNS # |  ized |  Tested |

----------------------------------------------------------------------------------------------

Shell |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Cone |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Cover |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Rings |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Rings |      SA-240 304 |   ... |                    ... |  S30400 |    No |      No |

Normalized is determined based on the UCS-66 material curve selection and Figure UCS-66.

Impact Tested is based on material selection and material data properties.

Element Pressures and MAWP (bars & mm.):

|        Design |         |           |             | Str. |    In |

Element Description |      Pressure |    Ext. |   Element |   Corrosion | Flg. | Creep |

or Type |  + Stat. head |  Press. |   M.A.W.P |   Allowance | Gov. | Range |

--------------------------------------------------------------------------------------------

Cylinder |         0.500 |    0.10 |    25.156 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     6.804 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     7.240 |      0.0000 |  N/A |    No |

Wld Flat |         0.500 |    0.10 |     2.124 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |    14.332 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     4.998 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     5.072 |      0.0000 |  N/A |    No |

Wld Flat |         0.500 |    0.10 |     1.653 |      0.0000 |  N/A |    No |

Cylinder |         0.500 |    0.10 |     9.055 |      0.0000 |  N/A |    No |

Stiffener Ring Specifications:

Elevation  mm.     Selected Type         User Description

0.00       Bar  39.0 x  12.                 Ring 1-1

1145.00       Bar  50.5 x  10.                 Ring 2-1

1724.00       Bar  49.0 x  10.                 Ring 5-1

1736.00       Bar  49.0 x  10.                 Ring 6-1

3605.00       Bar  47.8 x  10.                 Ring 6-2

4741.00       Bar  47.8 x  10.                 Ring 6-3

4846.00       Bar  47.0 x  10.                 Ring 7-1

6356.00       Bar  47.0 x  10.                 Ring 7-2

7469.00       Bar  26.0 x  14.                 Ring 8.1

Element Types and Properties:

Element |"To" Elev |  Element |  Nominal | Finished | Reqd Thk | Reqd Thk |   Long |   Circ |

|          |   Length |Thickness |Thickness | Internal | External |    Eff |    Eff |

Type |      mm. |      mm. |      mm. |      mm. |      mm. |      mm. |        |        |

------------------------------------------------------------------------------------------------

Cylinder |     50.0 |     50.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |   1150.0 |   1100.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |   1606.0 |    456.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Wld Flat |   1616.0 |     10.0 |     10.0 |     10.0 |      4.9 |      ... |   1.00 |   1.00 |

Cylinder |   1731.0 |    115.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |   4746.0 |   3015.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Cylinder |   7260.0 |   2514.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Wld Flat |   7274.0 |     14.0 |     14.0 |     14.0 |      7.7 |      ... |   1.00 |   1.00 |

Cylinder |   7474.0 |    200.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    20|          3 |    0.19212 |             0.1 |      12.103 |

20|  Ring|          3 |    1.27187 |             0.1 |     0.85662 |

Ring|    30|          3 |    0.43542 |             0.1 |     3.08702 |

30|    40|          3 |    0.99796 |             0.1 |     1.60685 |

40|    50|         10 |    No Calc |             0.1 |     No Calc |

50|  Ring|          3 |    0.36718 |             0.1 |     6.05541 |

Ring|    60|          3 |    0.20698 |             0.1 |     7.27422 |

60|  Ring|          3 |     0.2107 |             0.1 |     7.13086 |

Ring|  Ring|          3 |    1.66204 |             0.1 |     0.43479 |

Ring|  Ring|          3 |    1.72473 |             0.1 |     0.40042 |

Ring|    70|          3 |    0.59194 |             0.1 |     2.11938 |

70|  Ring|          3 |    0.64382 |             0.1 |      2.0579 |

Ring|  Ring|          3 |    2.02088 |             0.1 |     0.27114 |

Ring|    80|          3 |    1.63721 |             0.1 |     0.45972 |

80|    90|         14 |    No Calc |             0.1 |     No Calc |

90|  Ring|          3 |    0.61369 |             0.1 |     3.26313 |

Ring|   100|          3 |    0.32711 |             0.1 |      4.3309 |

-----------------------------------------------------------------------

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    20|              50 |          4811.25 |   0.00062658 |      9.67321 |

20|  Ring|         706.449 |          706.449 |      No Calc |      No Calc |

Ring|    30|         4.99191 |          4.99191 |      0.36588 |       20.652 |

30|    40|             456 |           6566.1 |      No Calc |      No Calc |

40|    50|         No Calc |          No Calc |      No Calc |      No Calc |

50|  Ring|             108 |          4021.26 |      No Calc |      No Calc |

Ring|    60|               7 |          487.092 |    0.0077134 |      17.0396 |

60|  Ring|         4.99249 |          4.99249 |      No Calc |      No Calc |

Ring|  Ring|         1374.53 |          1374.53 |     0.093369 |      17.0481 |

Ring|  Ring|         997.779 |          997.779 |      1.53939 |      18.2173 |

Ring|    70|         4.99741 |          4.99741 |       1.5006 |      19.1386 |

70|  Ring|             100 |          2398.86 |      No Calc |      No Calc |

Ring|  Ring|            1510 |          3984.95 |      2.40723 |      18.3065 |

Ring|    80|             904 |          3985.66 |      3.60856 |      18.3065 |

80|    90|         No Calc |          No Calc |      No Calc |      No Calc |

90|  Ring|             195 |          5498.92 |      No Calc |      No Calc |

Ring|   100|               5 |          216.544 |     0.052954 |      4.51783 |

--------------------------------------------------------------------------------

Factored Loads:

Un-Factored Loads:

Weights:

Fabricated - Bare  W/O Removable Internals                1106.2  kg.

Shop Test  - Fabricated + Water ( Full )                 10310.6  kg.

Shipping   - Fab. + Rem. Intls.+ Shipping App.            1106.2  kg.

Erected    - Fab. + Rem. Intls.+ Insul. (etc)             1106.2  kg.

Empty      - Fab. + Intls. + Details + Wghts.             1106.2  kg.

Operating  - Empty + Operating Liquid (No CA)             1106.2  kg.

Field Test - Empty Weight + Water (Full)                 10310.6  kg.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019