Table of Contents

Cover Sheet	2

Title Page	3

Rectves Analysis  : Rectangular-2	4

Vessel Results      Summary	13

MICROTEC

Rectangular Duct for Flash Dryer - Section 1

DESIGN CALCULATION

ASME Code Version     : 2017

Analysis Performed by : SL Client

Job File              : D:\2-PAYAM\1-Projects\Microtec Project\13-Microtec -Flash Dryer-5204\5204-Eng

Date of Analysis      : Feb 20,2026

PV Elite 2019 , January 2019

Note:

PV Elite performs all calculations internally in U.S. Customary Units

to remain compliant with the ASME Code and any built in assumptions in

the ASME Code formulae. Finalized results are converted to the user set of

selected units using conversion constants with adequate significant digits.

Input Echo, COMPONENT      1,         Description: Rectangular-2

Figure Number Analyzed                                    A4

Design Internal Pressure                   P          0.5000  bars

Design External Pressure                Pext          0.1000  bars

Design Temperature                      Temp        170.0000  C

VESSEL MATERIAL DATA:

Material Specification                            SA-240 316

Shell Allowable Stress at Design Temp      S        136.0672  N./mm^2

Shell Allowable Stress at Ambient         SA        137.9000  N./mm^2

Shell Yield Stress at Design Temperature  Sy        156.1065  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        750.0000  mm.

Minimum Thickness of Short-side Plates    t1          3.0000  mm.

Mid-side Joint Efficiency on Short-side    E          1.0000

Corner Joint Efficiency on Short-side     EC          1.0000

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h       1370.0000  mm.

Minimum Thickness of Long-side Plates     t2          3.0000  mm.

Mid-side Joint Efficiency on Long-side     E          1.0000

REINFORCEMENT MATERIAL DATA:

Reinforcement Material Specification              SA-240 316

Reinf Allowable Stress at Design Temp     Sr        136.0672  N./mm^2

Reinf Allowable Stress at Ambient         SA        137.9000  N./mm^2

Reinf Yield Stress at Design Temp         Sy        156.1065  N./mm^2

C-Factor for Reinforcement  (from UG-47)              2.1000

DELTA (Reinforcement Material Parameter)            485.0000  N./mm^2^0.5

SHORT-SIDE RECTANGULAR BEAM DATA:

Outside Distance from Outside of Vessel              50.0000  mm.

Width of Reinforcing Member                          10.0000  mm.

LONG-SIDE RECTANGULAR BEAM DATA:

Outside Distance from Outside of Vessel              50.0000  mm.

Width of Reinforcing Member                          10.0000  mm.

Rectangular Vessel Results, Item number     1, Desc: Rectangular-2

ASME Code, Section VIII, Division 1, 2017 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 1.000

Bending Ligament Efficiency [Eb]:

= 1.000

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 3.000 - 0.000 / 2

= 1.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 3.000 - 0.000 ) / 2

= -1.500 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 1.000

Bending Ligament Efficiency [Eb]:

= 1.000

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 3.000 - 0.000 / 2

= 1.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 3.000 - 0.000 ) / 2

= -1.500 mm.

Long-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 1.000

Bending Ligament Efficiency [Eb]:

= 1.000

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 3.000 - 0.000 / 2

= 1.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 3.000 - 0.000 ) / 2

= -1.500 mm.

Long-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 1.000

Bending Ligament Efficiency [Eb]:

= 1.000

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 3.000 - 0.000 / 2

= 1.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 3.000 - 0.000 ) / 2

= -1.500 mm.

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       1.000       1.000       1.500      -1.500

2       1.000       1.000       1.500      -1.500

Long-side  1       1.000       1.000       1.500      -1.500

2       1.000       1.000       1.500      -1.500

Effective Width of Shell Plate ( Section 13-8, Eq. (2) )

In Compression [w]:

= Min( Min( t1, t2 ) * Delta / sqrt( Sy ), p )

= Min( Min( 3.00 , 3.00 ) * 5840.83 / sqrt( 156.11 ), 65.00 )

= 65.00 mm.

In Tension [w]:

= 65.00 mm.

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0000 cm**4

Thickness  t2, I2  = 0.0000 cm**4

Effective Area of Reinforcement on Shell ( t * w ):

Short-side Ap = 1.9500 cm^2

Long-side  Ap = 1.9500 cm^2

Moment of Inertia of Effective Area of Reinforcement ( w * t**3 / 12 ):

Short-side Is = 0.0146 cm**4

Long-side  Il = 0.0146 cm**4

Moment of Inertia of Combined Reinforcement and Effective Width:

In Compression I11 = 20.2830 cm**4

I21 = 20.2830 cm**4

In Tension     I11 = 20.2830 cm**4

I21 = 20.2830 cm**4

Distance from Neutral Axis of Cross Section of Composite Section to

the Inside Surface of the Vessel (mm.):

Ci             Co

Short-side, in Compression         20.5647       -32.4353

in Tension             20.5647       -32.4353

Long-side,  in Compression         20.5647       -32.4353

in Tension             20.5647       -32.4353

Rectangular Vessel Reinforcement Parameters:

Alpha1 =     H1 / h1      = 0.5652

k(comp)= (I22/I11)*Alpha1 = 0.5652

k(tens)= (I22/I11)*Alpha1 = 0.5652

Membrane Stress Calculations per Section 13-8

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= p * h * p / ( 2 * ( A1 + p * t1 ))

= 0.50 * 1370.00 * 65.00 / ( 2 * ( 500.000 + 65.00 * 3.00 ))

= 3.20 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= p * h * p / ( 2 * ( A1 + p * t1 ))

= 0.50 * 1370.00 * 65.00 / ( 2 * ( 500.000 + 65.00 * 3.00 ))

= 3.20 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A[SmlA]:

= p * H * p / ( 2 * ( A2 + p * t2 ))

= 0.50 * 750.00 * 65.00 / ( 2 * ( 5.000 + 65.00 * 3.00 ))

= 1.75 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A[SmlA]:

= p * H * p / ( 2 * ( A2 + p * t2 ))

= 0.50 * 750.00 * 65.00 / ( 2 * ( 5.000 + 65.00 * 3.00 ))

= 1.75 N./mm^2

Membrane Stresses at Corner sections

Membrane Stress at Short side [Smsc]:

= p * h * p / ( 2 * ( A1 + p * t1 ))

= 0.50 * 1370.00 * 65.00 / ( 2 * ( 5.000 + 65.00 * 3.00 ))

= 3.20 N./mm^2

Membrane Stress at Long side [Smlc]:

= p * H * p / ( 2 * ( A2 + p * t2 ))

= 0.50 * 750.00 * 65.00 / ( 2 * ( 5.000 + 65.00 * 3.00 ))

= 1.75 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-8,

Equations (3) and (4). (N./mm^2) :

STRESS LOCATIONS                                Actual     Allowable

Short-side 1                                      3.20       136.07

Short-side 2                                      3.20       136.07

Short-side Corner                                 3.20       136.07

Long-side  1 at A                                 1.75       136.07

Long-side  2 at A                                 1.75       136.07

Long-side Corner                                  1.75       136.07

Bending Stress Calculations per Section 13-8

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * p * c / ( 24 * I11 ) * [ -3 * H2 + 2 * h2

* (( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.5 * 65.0 * 20.56 / ( 24 * 20.3 ) * [ -3 * 750.002 + 2 * 1370.002 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 15.71 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * p * c / ( 24 * I11 ) * [ -3 * H2 + 2 * h2

* (( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.5 * 65.0 * -32.44 / ( 24 * 20.3 ) * [ -3 * 750.002 + 2 * 1370.002 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -24.77 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * p * c / ( 12 * I11 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 12 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))

= 38.88 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * p * c / ( 12 * I11 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 12 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))

= -61.32 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * p * c / ( 24 * I11 ) * [ -3 * H2 + 2 * h2

* (( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.5 * 65.0 * 20.56 / ( 24 * 20.3 ) * [ -3 * 750.002 + 2 * 1370.002 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 15.71 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * p * c / ( 24 * I11 ) * [ -3 * H2 + 2 * h2

* (( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.5 * 65.0 * -32.44 / ( 24 * 20.3 ) * [ -3 * 750.002 + 2 * 1370.002 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -24.77 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * p * c / ( 12 * I11 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 12 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))

= 38.88 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * p * c / ( 12 * I11 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 12 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))

= -61.32 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * p * c / ( 24 * I21 ) * [ -3 + 2 *

(( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 24 * 20.28 ) * [ -3 + 2 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -38.44 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * p * c / ( 24 * I21 ) * [ -3 + 2 *

(( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 24 * 20.28 ) * [ -3 + 2 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 60.62 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * p * c / ( 12 * I21 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 24 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 38.88 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * p * c / ( 12 * I21 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 24 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -61.32 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * p * c / ( 24 * I21 ) * [ -3 + 2 *

(( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 24 * 20.28 ) * [ -3 + 2 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -38.44 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * p * c / ( 24 * I21 ) * [ -3 + 2 *

(( 1 + Alpha12 * k ) / ( 1 + k ))]

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 24 * 20.28 ) * [ -3 + 2 *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 60.62 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * p * c / ( 12 * I21 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * 20.56 / ( 24 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= 38.88 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * p * c / ( 12 * I21 ) *

(( 1 + Alpha12 * k ) / ( 1 + k ))

= 0.50 * 1370.002 * 65.00 * -32.44 / ( 24 * 20.28 ) *

(( 1 + 0.572 * 0.57 ) / ( 1 + 0.57 ))]

= -61.32 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-8,

Equations (5-8). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer      Allowable

Short-side 1 at N                    15.71      -24.77       104.07

at Q                    38.88      -61.32       104.07

Short-side 2 at N                    15.71      -24.77       104.07

at Q                    38.88      -61.32       104.07

Long-side  1 at M                   -38.44       60.62       104.07

at Q                    38.88      -61.32       104.07

Long-side  2 at M                   -38.44       60.62       104.07

at Q                    38.88      -61.32       104.07

Total Stress Calculations per Section 13-8

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 3.20 + 15.71

= 18.91 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 3.20 + -24.77

= -21.57 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Smsc + SbsQi

= 3.20 + 38.88

= 42.08 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Smsc + SbsQo

= 3.20 + -61.32

= -58.11 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 3.20 + 15.71

= 18.91 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 3.20 + -24.77

= -21.57 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Smsc + SbsQi

= 3.20 + 38.88

= 42.08 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Smsc + SbsQo

= 3.20 + -61.32

= -58.11 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= SmlA + SblMi

= 1.75 + -38.44

= -36.68 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= SmlA + SblMo

= 1.75 + 60.62

= 62.38 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Smlc + SblQi

= 1.75 + 38.88

= 40.63 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= Smlc + SblQo

= 1.75 + -61.32

= -59.56 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= SmlA + SblMi

= 1.75 + -38.44

= -36.68 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= SmlA + SblMo

= 1.75 + 60.62

= 62.38 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Smlc + SblQi

= 1.75 + 38.88

= 40.63 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= Smlc + SblQo

= 1.75 + -61.32

= -59.56 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-8,

Equations (9-12). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer      Allowable

Short-side 1 at N                    18.91      -21.57       104.07

at Q                    42.08      -58.11       104.07

Short-side 2 at N                    18.91      -21.57       104.07

at Q                    42.08      -58.11       104.07

Long-side  1 at M                   -36.68       62.38       104.07

at Q                    40.63      -59.56       104.07

Long-side  2 at M                   -36.68       62.38       104.07

at Q                    40.63      -59.56       104.07

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)              3.20   N./mm^2

High Stress Percentage                            2.35   %

M.A.W.P. for Membrane Stresses                   21.24   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -61.32   N./mm^2

High Stress Percentage                           58.92   %

M.A.W.P. for Bending Stresses                     0.85   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             62.38   N./mm^2

High Stress Percentage                           59.94   %

M.A.W.P. for Total Stresses                       0.83   bars

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Rectangular Vessel Results For Item 1 : A4

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)              3.20   N./mm^2

High Stress Percentage                            2.35   %

M.A.W.P. for Membrane Stresses                   21.24   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -61.32   N./mm^2

High Stress Percentage                           58.92   %

M.A.W.P. for Bending Stresses                     0.85   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             62.38   N./mm^2

High Stress Percentage                           59.94   %

M.A.W.P. for Total Stresses                       0.83   bars

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019