Table of Contents

Cover Sheet	2

Title Page	3

Warnings and Errors:	4

Input Echo:	5

XY Coordinate Calculations:	8

Internal Pressure Calculations:	9

External Pressure Calculations:	11

Element and Detail Weights:	13

Center of Gravity Calculation:	15

MDMT Summary:	16

Vessel Design Summary:	17

DESIGN CALCULATION

In Accordance with ASME Section VIII Division 1

ASME Code Version     : 2017

Analysis Performed by : SPLM Licensed User

Job File              : D:\2-PAYAM\1-PROJECTS\MICROTEC PROJECT\13-MICROT

Date of Analysis      : Feb 20,2026   5:10pm

PV Elite 2019 SP1, March 2019

Note:

PV Elite performs all calculations internally in Imperial Units

to remain compliant with the ASME Code and any built in assumptions

in the ASME Code formulas. The finalized results are reflected to show

the user's set of selected units.

Class From To : Basic Element Checks.

==========================================================================

Class From To: Check of Additional Element Data

==========================================================================

There were no geometry errors or warnings.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

PV Elite Vessel Analysis Program: Input Data

Design Internal Pressure (for Hydrotest)                 0.5  bars

Design Internal Temperature                            170.0  °C

Type of Hydrotest                              not Specified

Hydrotest Position                                Horizontal

Projection of Nozzle from Vessel Top                       0  mm.

Projection of Nozzle from Vessel Bottom                    0  mm.

Minimum Design Metal Temperature                       -28.9  °C

Type of Construction                                  Welded

Special Service                                         None

Degree of Radiography                                   RT-1

Use Higher Longitudinal Stresses (Flag)                    Y

Select t for Internal Pressure (Flag)                      N

Select t for External Pressure (Flag)                      N

Select t for Axial Stress (Flag)                           N

Select Location for Stiff. Rings (Flag)                    N

Consider Vortex Shedding                                   N

Perform a Corroded Hydrotest                               N

Load Case 1                                   NP+EW+WI+FW+BW

Load Case 2                                   NP+EW+EE+FS+BS

Load Case 3                                   NP+OW+WI+FW+BW

Load Case 4                                   NP+OW+EQ+FS+BS

Load Case 5                                         NP+HW+HI

Load Case 6                                         NP+HW+HE

Load Case 7                                   IP+OW+WI+FW+BW

Load Case 8                                   IP+OW+EQ+FS+BS

Load Case 9                                   EP+OW+WI+FW+BW

Load Case 10                                  EP+OW+EQ+FS+BS

Load Case 11                                        HP+HW+HI

Load Case 12                                        HP+HW+HE

Load Case 13                                        IP+WE+EW

Load Case 14                                        IP+WF+CW

Load Case 15                                        IP+VO+OW

Load Case 16                                        IP+VE+EW

Load Case 17                                        NP+VO+OW

Load Case 18                                     FS+BS+IP+OW

Load Case 19                                     FS+BS+EP+OW

Wind Design Code                                   ASCE-7 93

Basic Wind Speed                     [V]              112.65  Km/hr

Surface Roughness Category                   C: Open Terrain

Importance Factor                                        1.0

Type of Surface                            Moderately Smooth

Base Elevation                                             0  mm.

Percent Wind for Hydrotest                              33.0

Using User defined Wind Press. Vs Elev.                    N

Damping Factor (Beta) for Wind (Ope)                  0.0100

Damping Factor (Beta) for Wind (Empty)                0.0000

Damping Factor (Beta) for Wind (Filled)               0.0000

Seismic Design Code                                   UBC 94

UBC Seismic Zone (1=1,2=2a,3=2b,4=3,5=4)               0.000

UBC Importance Factor                                  1.000

UBC Soil Type                                          S1

UBC Horizontal Force Factor                            3.000

UBC Percent Seismic for Hydrotest                      0.000

Design Pressure + Static Head                              Y

Consider MAP New and Cold in Noz. Design                   N

Consider External Loads for Nozzle Des.                    Y

Use ASME VIII-1 Appendix 1-9                               N

Material Database Year        Current w/Addenda or Code Year

Configuration Directives:

Do not use Nozzle MDMT Interpretation VIII-1 01-37        No

Use Table G instead of exact equation for "A"            Yes

Shell Head Joints are Tapered                            Yes

Compute "K" in corroded condition                        Yes

Use Code Case 2286                                        No

Use the MAWP to compute the MDMT                         Yes

For thickness ratios <= 0.35, MDMT will be -155F (-104C) Yes

For PWHT & P1 Materials the MDMT can be < -55F (-48C)     No

Using Metric Material Databases, ASME II D                No

Calculate B31.3 type stress for Nozzles with Loads       Yes

Reduce the MDMT due to lower membrane stress             Yes

Consider Longitudinal Stress in MDMT calcs. (Div. 1)      No

Complete Listing of Vessel Elements and Details:

Element From Node                                         10

Element To Node                                           20

Element Type                                        Cylinder

Description

Distance "FROM" to "TO"                                   50  mm.

Inside Diameter                                          300  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Allowable Stress, Ambient                          137.9  N./mm²

Allowable Stress, Operating                       136.07  N./mm²

Allowable Stress, Hydrotest                       179.27  N./mm²

Material Density                                0.008027  kg./cm³

P Number Thickness                                     0  mm.

Yield Stress, Operating                           156.11  N./mm²

External Pressure Chart Name                        HA-2

UNS Number                                        S31600

Product Form                                       Plate

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Weld is pre-Heated                                        No

Element From Node                                     10

Detail Type                                         Ring

Detail ID                                  Ring:[1 of 1]

Dist. from "FROM" Node / Offset dist                   0  mm.

Inside Diameter of Ring                              306  mm.

Thickness of Ring                                     14  mm.

Outside Diameter of Ring                             370  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

--------------------------------------------------------------------

Element From Node                                         20

Element To Node                                           40

Element Type                                        Conical

Description

Distance "FROM" to "TO"                                 2000  mm.

Inside Diameter                                          300  mm.

Element Thickness                                          3  mm.

Internal Corrosion Allowance                               0  mm.

Nominal Thickness                                          3  mm.

External Corrosion Allowance                               0  mm.

Design Internal Pressure                                 0.5  bars

Design Temperature Internal Pressure                     170  °C

Design External Pressure                                 0.1  bars

Design Temperature External Pressure                      50  °C

Effective Diameter Multiplier                            1.2

Material Name                                     SA-240 316

Efficiency, Longitudinal Seam                            1.0

Efficiency, Circumferential Seam                         1.0

Cone Diameter at "To" End                               1920  mm.

Design Length of Cone                                   2000  mm.

Half Apex Angle of Cone                            22.047947  degrees

Toriconical (Y/N)                                          N

Weld is pre-Heated                                        No

Element From Node                                     20

Detail Type                                         Ring

Detail ID                                      Flange -1

Dist. from "FROM" Node / Offset dist                1983  mm.

Inside Diameter of Ring                           1912.2  mm.

Thickness of Ring                                     12  mm.

Outside Diameter of Ring                          2020.9  mm.

Material Name                                 SA-240 304

Height of Section Ring                                 0  mm.

Using Custom Stiffener Section                        No

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

XY Coordinate Calculations:

|    |            |            |             |             |

From     | To |  X (Horiz.)|  Y (Vert.) |  DX (Horiz.)|  DY (Vert.) |

|    |        mm. |        mm. |         mm. |         mm. |

----------------------------------------------------------------------

10     |  20|        ... |         50 |         ... |          50 |

20     |  40|        ... |       2050 |         ... |        2000 |

----------------------------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element Thickness, Pressure, Diameter and Allowable Stress :

|    | Int. Press |    Nominal | Total Corr |    Element |  Allowable |

From      | To | + Liq. Hd  |  Thickness |  Allowance |   Diameter |  Stress(SE)|

|    |       bars |        mm. |        mm. |        mm. |     N./mm² |

----------------------------------------------------------------------------------

10      |  20|        0.5 |          3 |        ... |        300 |     136.07 |

20      |  40|        0.5 |          3 |        ... |       1920 |     136.07 |

----------------------------------------------------------------------------------

Element Required Thickness and MAWP :

|    |     Design |   M.A.W.P. |     M.A.P. |    Minimum |   Required |

From      | To |   Pressure |   Corroded | New & Cold |  Thickness |  Thickness |

|    |       bars |       bars |       bars |        mm. |        mm. |

----------------------------------------------------------------------------------

10      |  20|        0.5 |    26.8892 |    27.2514 |          3 |        1.5 |

20      |  40|        0.5 |    3.93408 |    3.98707 |          3 |        1.5 |

----------------------------------------------------------------------------------

Minimum                          3.934        3.987

MAWP: 3.934 bars, limited by: Cone.

Internal Pressure Calculation Results :

ASME Code, Section VIII Division 1, 2017

Cylindrical Shell From 10 To 20 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*R)/(S*E-0.6*P) per UG-27 (c)(1)

= (0.5*150.0)/(136.07*1.0-0.6*0.5)

= 0.0551 + 0.0000 = 0.0551 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (136.07*1.0*3.0)/(150.0+0.6*3.0)

= 26.889 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (S*E*t)/(R+0.6*t) per UG-27 (c)(1)

= (137.9*1.0*3.0)/(150.0+0.6*3.0)

= 27.251 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(R+0.6*t))/(E*t)

= (0.5*(150.0+0.6*3.0))/(1.0*3.0)

= 2.530 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.990 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Conical Section From 20 To 40 SA-240 316 at 170 °C

Material UNS Number:    S31600

Required Thickness due to Internal Pressure [tr]:

= (P*D)/(2*cos(a)*(S*E-0.6*P)) per Appendix 1-4 (e)

= (0.5*1920.0)/(2*0.9269*(136.07*1.0-0.6*0.5))

= 0.3807 + 0.0000 = 0.3807 mm.

Note: The thickness required was less than the Code Minimum, therefore

the Code Minimum value of 1.5000 mm. per UG-16 will be used.

Max. Allowable Working Pressure at given Thickness, corroded [MAWP]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*136.07*1.0*3.0*0.927)/(1920.0+1.2*3.0*0.927)

= 3.934 bars

Maximum Allowable Pressure, New and Cold [MAPNC]:

= (2*S*E*t*cos(a))/(D+1.2*t*cos(a)) per App 1-4(e)

= (2*137.9*1.0*3.0*0.9269)/(1920.0+1.2*3.0*0.9269)

= 3.987 bars

Actual stress at given pressure and thickness, corroded [Sact]:

= (P*(D+1.2*t*cos(a)))/(2*E*t*cos(a))

= (0.5*(1920.0+1.2*3.0*0.9269))/(2*1.0*3.0*0.9269)

= 17.293 N./mm²

% Elongation per Table UG-79-1 (50*tnom/Rf)*(1-Rf/Ro)  0.989 %

Note: Please Check Requirements of Table UHA-44 for Elongation limits.

Note: The Pressure at the Large Diameter is used in the TR calculation.

SA-240 316, Min Metal Temp without impact per UHA-51:  -196 °C

Elements Suitable for Internal Pressure.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

External Pressure Calculation Results :

External Pressure Calculations:

|      |    Section |    Outside |  Corroded |    Factor  |     Factor |

From|   To |     Length |   Diameter | Thickness |       A    |        B   |

|      |        mm. |        mm. |       mm. |            |     N./mm² |

-----------------------------------------------------------------------------

10|  Ring|    No Calc |        ... |         3 |    No Calc |    No Calc |

Ring|    20|         50 |        306 |         3 |    0.01054 |    99.0623 |

20|  Ring|    1150.16 |     1912.7 |         3 | 0.00012505 |    12.0049 |

Ring|    40|    16.9392 |    1926.47 |         3 |  0.0021617 |    81.3491 |

-----------------------------------------------------------------------------

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    20|          3 |    0.18486 |             0.1 |     12.9486 |

20|  Ring|          3 |    2.15271 |             0.1 |     0.23268 |

Ring|    40|          3 |    0.75209 |             0.1 |     1.56547 |

-----------------------------------------------------------------------

Minimum                                                      0.233

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    20|              50 |          5133.63 |   0.00051345 |      6.33378 |

20|  Ring|         1150.16 |          1150.16 |      No Calc |      No Calc |

Ring|    40|         16.9392 |          16.9392 |      2.92476 |      30.9459 |

--------------------------------------------------------------------------------

Elements Suitable for External Pressure.

ASME Code, Section VIII Division 1, 2017

Cone From 20 to Flange -1 Ext. Chart: HA-2 at 50 °C

Elastic Modulus from Chart: HA-2 at 50 °C    :   0.192E+09 KPa.

Results for Maximum Allowable External Pressure (MAEP):

Tca         OD        SLEN       D/t       L/D   Factor A        B

2.781    1912.70     1150.16    687.87    0.6013  0.0001251    12.00

EMAP = (4*B)/(3*(D/t)) = (4*12.0049)/(3*687.8719) = 0.2327 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Results for Required Thickness (Tca):

Tca         OD        SLEN       D/t       L/D   Factor A        B

1.995    1912.70     1150.16    958.61    0.6013  0.0000749     7.19

EMAP = (4*B)/(3*(D/t)) = (4*7.1904)/(3*958.6123) = 0.1 bars

Note: The cone thickness used in the calculation has been modified

per UG-33(f), te = t * cos(alpha).

Cone From Flange -1 to the end: Ext. Chart: HA-2 at 50 °C

Stiffening Ring Calcs for : Ring:[1 of 1] , SA-240 304 , Bar Ring: 32 x 14 mm.

Effective Length of Shell:                            33 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             1.000          1.5000           1.500

Ring :             4.480         19.0000          85.120

Total:             5.480                          86.620

--------------------------------------------------------

Centroid of Ring plus Shell:                          16 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.007         14.3070         204.659

Ring :             3.823         -3.1930          45.676

Total:             3.830                         250.335

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:          6 cm**4

Required Stress in Ring plus Shell      Breq        0.11 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000011

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (306.0²*25.0(3.0+4.48/25.0)0.000001)/10.9

= 0 cm**4

Stiffening Ring Calcs for : Flange -1 , SA-240 304 , Bar Ring: 54 x 12 mm.

Effective Length of Shell:                            83 mm.

Area (cm²)   Distance (mm.)    Area*Dist

--------------------------------------------------------

Shell:             2.499          1.5000           3.749

Ring :             6.523         30.1800         196.870

Total:             9.023                         200.619

--------------------------------------------------------

Centroid of Ring plus Shell:                          22 mm.

Inertia        Distance         A*Dist²

--------------------------------------------------------

Shell:             0.019         20.7351        1074.620

Ring :            16.063         -7.9449         411.755

Total:            16.082                        1486.376

--------------------------------------------------------

Available Moment of Inertia, Ring plus Shell:         31 cm**4

Required Stress in Ring plus Shell      Breq        3.48 N./mm²

Required Strain in Ring plus Shell      Areq   0.0000363

Required Moment of Inertia, Ring plus Shell:

= ( OD² * Slen(Tca + Aring/Slen)Areq )/10.9

= (1912.2301²*583.5508(3.0+6.5232/583.5508)0.000036)/10.9

= 3 cm**4

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Element and Detail Weights:

|    |    Element |    Element |  Corroded |   Corroded | Extra due  |

From| To | Metal Wgt. | ID  Volume |Metal Wgt. |  ID Volume | Misc %     |

|    |        kg. |        Cm. |       kg. |        Cm. |        kg. |

---------------------------------------------------------------------------

10|  20|    1.14618 |    3534.92 |   1.14618 |    3534.92 |        ... |

20|  40|    181.738 |    2279319 |   181.738 |    2279319 |        ... |

---------------------------------------------------------------------------

Total |        182 | 2282854.50 |       182 | 2282854.50 |          0 |

Weight of Details:

|    |  Weight of |  X Offset, | Y Offset, |

From|Type|     Detail | Dtl. Cent. |Dtl. Cent. |   Description

|    |        kg. |        mm. |       mm. |

-------------------------------------------------

10|Ring|    3.81869 |        ... |       ... |   Ring:[1 of 1]

20|Ring|    32.3515 |        ... |      1983 |   Flange -1

-------------------------------------------------

Total Weight of Each Detail Type:

Stiffeners                         36.2

-------------------------------------------

Sum of the Detail Weights          36.2 kg.

Weight Summation Results: (kg.)

| Fabricated |  Shop Test |   Shipping |    Erected |      Empty |  Operating |

---------------------------------------------------------------------------------------------

Main Elements |      182.9 |      182.9 |      182.9 |      182.9 |      182.9 |      182.9 |

Stif. Rings |       36.2 |       36.2 |       36.2 |       36.2 |       36.2 |       36.2 |

Test Liquid |        ... |     2281.5 |        ... |        ... |        ... |        ... |

---------------------------------------------------------------------------------------------

Totals |      219.1 |     2500.5 |      219.1 |      219.1 |      219.1 |      219.1 |

Weight Summary:

Fabricated Wt.  - Bare Weight without Removable Internals                   219.1 kg.

Shop Test Wt.   - Fabricated Weight + Water ( Full )                       2500.5 kg.

Shipping Wt.    - Fab. Weight + removable Intls.+ Shipping App.             219.1 kg.

Erected  Wt.    - Fab. Wt + or - loose items (trays,platforms etc.)         219.1 kg.

Ope. Wt. no Liq - Fab. Weight + Internals. + Details + Weights              219.1 kg.

Operating Wt.   - Empty Weight + Operating Liq. Uncorroded                  219.1 kg.

Field Test Wt.  - Empty Weight + Water (Full)                              2500.5 kg.

Mass of the Upper 1/3 of the Vertical Vessel                                 94.4 kg.

Outside Surface Areas of Elements:

|    |        Surface |

From| To |           Area |

|    |            cm² |

----------------------------

10|  20|        480.664 |

20|  40|        75684.9 |

----------------------------

Total         76165.602 cm²

Element and Detail Weights:

| To |  Total Ele.| Total. Ele.|Total. Ele.|  Total Dtl.| Oper. Wgt. |

From| To |  Empty Wgt.|  Oper. Wgt.|Hydro. Wgt.| Offset Mom.| No Liquid  |

|    |        kg. |        kg. |       kg. |      Kg-m. |        kg. |

---------------------------------------------------------------------------

10|  20|    4.96487 |    4.96487 |   4.96487 |        ... |    4.96487 |

20|  40|     214.09 |     214.09 |    214.09 |        ... |     214.09 |

---------------------------------------------------------------------------

Cumulative Vessel Weight

|    | Cumulative Ope | Cumulative | Cumulative  |

From| To | Wgt. No Liquid | Oper. Wgt. | Hydro. Wgt. |

|    |            kg. |        kg. |         kg. |

-------------------------------------------------------

10|  20|        219.055 |    219.055 |     219.055 |

20|  40|         214.09 |     214.09 |      214.09 |

-------------------------------------------------------

Note: The cumulative operating weights no liquid in the column above

are the cumulative operating weights minus the operating liquid

weight minus any weights absent in the empty condition.

Cumulative Vessel Moment

|    | Cumulative | Cumulative |Cumulative |

From| To | Empty Mom. | Oper. Mom. |Hydro. Mom.|

|    |      Kg-m. |      Kg-m. |     Kg-m. |

-------------------------------------------------

10|  20|        ... |        ... |       ... |

20|  40|        ... |        ... |       ... |

-------------------------------------------------

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Shop/Field Installation Options :

Note : The CG is computed from the first Element From Node

Center of Gravity of Stiffening Rings                      1818.365 mm.

Center of Gravity of Bare Shell New and Cold               1285.295 mm.

Center of Gravity of Bare Shell Corroded                   1285.295 mm.

Vessel CG in the Operating Condition                       1373.315 mm.

Vessel CG in the Fabricated (Shop/Empty) Condition         1373.315 mm.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

Minimum Design Metal Temperature Results Summary :

Curve  Basic  Reduced  UG-20(f)  Thickness     Gov      E*   PWHT

Description               MDMT     MDMT      MDMT      ratio     Thk           reqd

Notes           °C       °C        °C                mm.

-----------------------------------------------------------------------------------

[16]         -196

[16]         -196

-----------------------------------------------------------------------------------

Warmest MDMT:             -196

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Notes:

[  ! ] - This was an impact tested material.

[  1] - Governing Nozzle Weld.

[  4] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-c).

[  5] - ANSI Flange MDMT Calcs; Thickness ratio per UCS-66(b)(1)(-b).

[  6] - MDMT Calculations at the Shell/Head Joint.

[  7] - MDMT Calculations for the Straight Flange.

[  8] - Cylinder/Cone/Flange Junction MDMT.

[  9] - Calculations in the Spherical Portion of the Head.

[10] - Calculations in the Knuckle Portion of the Head.

[11] - Calculated (Body Flange) Flange MDMT.

[12] - Calculated Flat Head MDMT per UCS-66.3

[13] - Tubesheet MDMT, shell side, if applicable

[14] - Tubesheet MDMT, tube side, if applicable

[15] - Nozzle Material

[16] - Shell or Head Material

[17] - Impact Testing required

[18] - Impact Testing not required, see UCS-66(b)(3)

[19] - Select a valid hydrotest type to get the UG-20(f) exemption

[20] - Cylinder/Cone Junction MDMT based on Longitudinal Stress considerations

[21] - Bolting Material

UG-84(b)(2) was not considered.

UCS-66(g) was not considered.

UCS-66(i) was not considered.

Notes:

Impact test temps were not entered in and not considered in the analysis.

UCS-66(i) applies to impact tested materials not by specification and

UCS-66(g) applies to materials impact tested per UG-84.1 General Note (c).

The Basic MDMT includes the (30F) PWHT credit if applicable.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019

ASME Code, Section VIII Division 1, 2017

Diameter Spec : 300.000 mm. ID

Vessel Design Length, Tangent to Tangent           2050.00  mm.

Distance of Bottom Tangent above Grade               50.00  mm.

Distance of Base above Grade                          0.00  mm.

Specified Datum Line Distance                         0.00  mm.

Internal Design Temperature                            170  °C

Internal Design Pressure                             0.500  bars

External Design Temperature                             50  °C

External Design Pressure                             0.100  bars

Maximum Allowable Working Pressure                   3.934  bars

External Max. Allowable Working Pressure             0.233  bars

Hydrostatic Test Pressure                            0.000  bars

Required Minimum Design Metal Temperature            -28.9  °C

Warmest Computed Minimum Design Metal Temperature   -196.0  °C

Wind Design Code                                   ASCE-93

Earthquake Design Code                              UBC-94

Materials of Construction:

Component |                 |       |                        |         |Normal |  Impact |

Type |        Material | Class |              Thickness |   UNS # |  ized |  Tested |

----------------------------------------------------------------------------------------------

Shell |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Cone |      SA-240 316 |   ... |                    ... |  S31600 |    No |      No |

Rings |      SA-240 304 |   ... |                    ... |  S30400 |    No |      No |

Normalized is determined based on the UCS-66 material curve selection and Figure UCS-66.

Impact Tested is based on material selection and material data properties.

Element Pressures and MAWP (bars & mm.):

|        Design |         |           |             | Str. |    In |

Element Description |      Pressure |    Ext. |   Element |   Corrosion | Flg. | Creep |

or Type |  + Stat. head |  Press. |   M.A.W.P |   Allowance | Gov. | Range |

--------------------------------------------------------------------------------------------

Cylinder |         0.500 |    0.10 |    26.889 |      0.0000 |  N/A |    No |

Conical |         0.500 |    0.10 |     3.934 |      0.0000 |  N/A |    No |

Stiffener Ring Specifications:

Elevation  mm.     Selected Type         User Description

0.00       Bar  32.0 x  14.            Ring:[1 of 1]

2033.00       Bar  54.4 x  12.                Flange -1

Element Types and Properties:

Element |"To" Elev |  Element |  Nominal | Finished | Reqd Thk | Reqd Thk |   Long |   Circ |

|          |   Length |Thickness |Thickness | Internal | External |    Eff |    Eff |

Type |      mm. |      mm. |      mm. |      mm. |      mm. |      mm. |        |        |

------------------------------------------------------------------------------------------------

Cylinder |     50.0 |     50.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

Conical  |   2050.0 |   2000.0 |      3.0 |      3.0 |      1.5 |      ... |   1.00 |   1.00 |

External Pressure Calculations:

|      |   External |   External |        External |    External |

From|   To |  Actual T. | Required T.| Design Pressure |    M.A.W.P. |

|      |        mm. |        mm. |            bars |        bars |

-----------------------------------------------------------------------

10|  Ring|          3 |    No Calc |             0.1 |     No Calc |

Ring|    20|          3 |    0.18486 |             0.1 |     12.9486 |

20|  Ring|          3 |    2.15271 |             0.1 |     0.23268 |

Ring|    40|          3 |    0.75209 |             0.1 |     1.56547 |

-----------------------------------------------------------------------

External Pressure Calculations:

|      |   Actual Length | Allowable Length | Ring Inertia | Ring Inertia |

From|   To | Bet. Stiffeners |  Bet. Stiffeners |     Required |    Available |

|      |             mm. |              mm. |        cm**4 |        cm**4 |

--------------------------------------------------------------------------------

10|  Ring|         No Calc |          No Calc |      No Calc |      No Calc |

Ring|    20|              50 |          5133.63 |   0.00051345 |      6.33378 |

20|  Ring|         1150.16 |          1150.16 |      No Calc |      No Calc |

Ring|    40|         16.9392 |          16.9392 |      2.92476 |      30.9459 |

--------------------------------------------------------------------------------

Factored Loads:

Un-Factored Loads:

Weights:

Fabricated - Bare  W/O Removable Internals                 219.1  kg.

Shop Test  - Fabricated + Water ( Full )                  2500.5  kg.

Shipping   - Fab. + Rem. Intls.+ Shipping App.             219.1  kg.

Erected    - Fab. + Rem. Intls.+ Insul. (etc)              219.1  kg.

Empty      - Fab. + Intls. + Details + Wghts.              219.1  kg.

Operating  - Empty + Operating Liquid (No CA)              219.1  kg.

Field Test - Empty Weight + Water (Full)                  2500.5  kg.

PV Elite is a trademark of Intergraph CADWorx & Analysis Solutions, Inc. 2019